Yaw Rate Sensor Logic

How to calculate when you’ve made a 90 turn?  Use accumulation of counts method.

Gyro output varies from 0.25V to 4.75V for rotational velocity of   75/sec.

This connects to analog input #1 or SENSOR1 and will give a count of approx 13-241  (we don’t get the full 5.0V swing to give a 0-254 count).  Total count range is 228.  Half is 114 counts.

- for a positive (right) turn, one count at SENSOR1 = +75 /sec  114 (max range)  positive counts = 0.658/sec/count.

Over the sampling time of 0.026 sec, the heading change is:

0.658/sec/count   X    0.026 sec =  0.017/count.

Therefore, to do a 90 right turn we need to accumulate   90  0.017/count  = 5262 counts.  (A count being the number input from SENSOR1, e.g. a count of 184 for a medium speed right turn, and adjusted for the 127 being neutral or no turn velocity.)

Since we can’t count above 255 with a single byte variable, we will have to use a 16 bit WORD variable named  yaw_ct  to accumulate the total count.

So, to do a 90 right turn,

Initialize all counters

Start turn  (L_DR = 254, R_DR = 0)

Increment a counter, new_ct ,  for each sample loop.  (use SELECT CASE to handle the < > 127), we want counts over 127, representing right turn, to add, and  less than 127, representing left turn, to subtract.  Accumulate each sample times counts into the variable yaw_ct.    When yaw_ct gets to a count of 5262, we should have turned a total of approximately 90.

So end the turn.

Sample code (that actually works in the emulator):

'  ==================   here is the test gyro reading code =====================================

drive_L = 254			' start right turn

drive_R = 1

select sensor1			'select yaw rate sensor

case 0 to 127		' turning left

new_ct = 127-sensor1

yaw_ct = yaw_ct - new_ct

case 128 to 255		' turning right

new_ct = sensor1 -127

yaw_ct = yaw_ct + new_ct

endselect

if yaw_ct > 5262  then	‘count has reached 5262 and passed 90

drive_L = 127  		‘stop turn

drive_R = 127

endif

debug ?yaw_ct		'display count value in debug screen

'  ========================================================================================

To do the same thing for a 90 left turn, do the same thing, except initialize   yaw_ct  at a higher number (say 10,000) and then let it decrement down by subtracting  new_ct  until it decrements down by 5262 counts, then stop the turn.

I tested this in the emulator and by varying the SENSOR1 slider can represent a turn.  We can watch the counter accumulate counts, both plus and minus for left and right, respectively, and when the count reached 5262, the motors went to stop (127).

We also today wired the yaw rate sensor into the analog input of a RC and watched the input on a dashboard program on the OI.  We could watch the dashboard indication of sensor1 go from 0-255 by turning the sensor left and right.  So it seems to work as it should.

I think we’re almost there.  We just need to work this technique code into the autonomous loop code for the two 90 turns, and we’re there.   (Vince, in this case “we” means “you”.)

Added 4/10/03 at the Nats in Houston:

Well, the code works.  Makes very consistent 90 degree turns.  We can tune our autonomous code to go up the center of the ramp to get the stacks, or we can tune it to go up our side of the ramp if our alliance partner is going up his side.  We used this technique with both of our alliance partners to win the 2003 Lone Star Regional.  We generally hit the stacks within a second of each other.

Now to see if we can carry this through the National competition.

Bruce Coons  Engineer  Team 647 CyberWolves

B. Coons										11 FEB 2003