# Steven Forbes

November 19, 2009

AME 452, Exam 2

## Body Coordinate Analysis of Single DOF Robotic Arm

# Background

The FIRST Robotics Competition is a high school level robotics contest put on annually by the organization FIRST (For Inspiration and Recognition of Science and Technology) and features a varying challenge from year to year.  2007’s challenge, dubbed “Rack ‘n’ Roll”, required that teams place inner tubes onto movable racks of different heights to score points.  The most successful teams devised robots capable of scoring quickly on the rack at any height.

One of the primary design constraints was the need for the robot to begin in a confined volume (28” x 38” x 48”) and still reach the highest rack, which was just high enough to be out of reach of a simple single jointed arm.  While most teams devised manipulators with multiple actuators to complete the task, a team from Sterling Heights, Michigan came up with a single degree of freedom (DOF) linkage system that could reach the rack and still fit in the required starting volume (fig. 1).  This resulted in an easier to control system requiring only a single actuator to complete the task.

Figure 1: Sterling Heights reaching over a defending opponent after scoring a game piece.

# System Description

The entire mechanism (fig. 2) consists of 5 bodies, as well as one ‘ground’ link (which in this case is the robot itself).  The system can best be described as two four-bar mechanisms in series connecting the robot base to the end of the manipulator.  An inversion in the first four-bar (ABQP) results in the entire linkage becoming fairly straight when in the extended position.  This gives the mechanism more reaching distance than a simple single jointed arm.  The second four-bar mechanism (BCDE) serves as a means of keeping the manipulator end in the desired orientation, which happens to be parallel to link 1 for this particular application.

In addition to these links, there is a gas spring that helps counter the weight of the arm.  This takes much of the load off of the motor, meaning that it can be geared to go faster (since less force is required to move the arm).  Because it is a gas spring, the force provided by the spring to support the arm is fairly constant regardless of the length of the spring.  This means that length calculations can be neglected when computing the force of the spring.  Table 1 lists the initial positions for the points and bodies.

Figure 1. Mechanism model

Table 1. Initial Positions of Points and Bodies

| Points | x | y | Bodies | x | y | phi |
| --- | --- | --- | --- | --- | --- | --- |
| A | 0.80 | 1.08 | 1 | 0.45 | 1.00 | 0 |
| B | 0.80 | 1.00 | 2 | 0.40 | 0.99 | 0.221 |
| C | 0.90 | 1.00 | 3 | 0.70 | 0.60 | 1.326 |
| D | 0.60 | 0.20 | 4 | 0.80 | 0.60 | 1.326 |
| E | 0.70 | 0.20 | 5 | 0.70 | 0.10 | 0 |
| F | 0.90 | 0.00 |  |  |  |  |
| O | 0.00 | 0.00 |  |  |  |  |
| P | 0.00 | 1.00 |  |  |  |  |
| Q | 0.00 | 0.90 |  |  |  |  |
| R | 0.10 | 0.70 |  |  |  |  |

The 5 bodies of the mechanism each have three degrees of freedom (horizontal translation, vertical translation, and rotation).  This gives a total of 15 degrees of freedom for all of the bodies.

There are a total of 7 joints that make up the mechanism; A,B,C,D,E,P, and Q.  Points F and G are secondary and not used in the formulation of the linkage, except to account for additional forces due to the gas spring and any mass that may be attached to the end of the arm.  Each joint removes 2 degrees of freedom from the system (14 in all), so the total system has only one degree of freedom.  In addition, this means that the Jacobian matrix of the system will consist of 14 rows and 15 columns (the construction of which is fairly straightforward, given that all joints in the system are revolute joints).

The mass and moment of inertia of each body is listed in table 2.  These values correspond directly to the mass matrix, which is a 15 by 15 matrix for the system.  Unlike previous the joint kinematic formulation that was previously used on this same system, masses of the bodies do not need to be split up into various points on the body (so long as the origin of the body’s coordinates lies at it’s center of mass).  The resulting mass matrix therefore only has diagonal terms and is easy to construct.

### Table 2. Mass and Moment of Inertia of Bodies

| Body | Mass (kg) | Moment of inertia (kg*m2) |
| --- | --- | --- |
| 1 | 2.0 | 0.135 |
| 2 | 1.0 | 0.056 |
| 3 | 1.0 | 0.068 |
| 4 | 1.0 | 0.057 |
| 5 | 2.0 | 0.017 |

# Program Output

The program was written to construct the phi matrix, jacobian matrix, right hand side array, mass matrix, and force matrix for the system.  The positions and values in the above figures and tables were used in the formulation.  Note that in the program output, all values in the phi matrix are close to zero, implying that the point and length measurements were correct and the program is working properly.  Printouts of the final output of the program can be found at the end of this report.

The mechanism was given initial velocities of zero for all bodies, meaning the mechanism should be at rest.  If a forward kinematic analysis is done on the model, we can see the effect of the gas spring on the mechanism.  A stiffer gas spring will result in the mechanism coming to rest at a different position.  This can be very useful if designing such a mechanism; different geometries can be easily constructed and appropriately sized springs determined for each.  An optimal system would require little force to move the linkage at any position, since less work would be required of the driving motor.

A kinematic analysis was also done with the body coordinate formulation, the results of which can be seen in figure 3.  The program ‘BC_kin_record.m’ records the x and y positions of all points and center of masses at each time interval (with body 1 being the driven link).  The paths of the joints and linkages can then be plotted (using ‘BC_plot.m’), giving useful information about the motion of the mechanism.

Figure 3

Constraints:

1.0e-003 *

0.0205

0.0754

-0.0763

-0.3011

-0.0763

-0.3011

-0.0763

-0.3011

-0.0763

-0.3011

0

0

0.0000

-0.0002

Jacobain Matrix:

0         0         0   -1.0000         0    0.0900    1.0000         0   -0.4801         0         0         0         0         0         0

0         0         0         0   -1.0000   -0.4000         0    1.0000    0.1000         0         0         0         0         0         0

-1.0000         0         0         0         0         0    1.0000         0   -0.3997         0         0         0         0         0         0

0   -1.0000   -0.3500         0         0         0         0    1.0000    0.0999         0         0         0         0         0         0

-1.0000         0         0         0         0         0         0         0         0    1.0000         0   -0.3997         0         0         0

0   -1.0000   -0.4500         0         0         0         0         0         0         0    1.0000    0.0999         0         0         0

0         0         0         0         0         0   -1.0000         0   -0.3997         0         0         0    1.0000         0   -0.1000

0         0         0         0         0         0         0   -1.0000    0.0999         0         0         0         0    1.0000   -0.1000

0         0         0         0         0         0         0         0         0   -1.0000         0   -0.3997    1.0000         0   -0.1000

0         0         0         0         0         0         0         0         0         0   -1.0000    0.0999         0    1.0000         0

-1.0000         0         0         0         0         0         0         0         0         0         0         0         0         0         0

0   -1.0000    0.4500         0         0         0         0         0         0         0         0         0         0         0         0

0         0         0   -1.0000         0   -0.0900         0         0         0         0         0         0         0         0         0

0         0         0         0   -1.0000    0.4000         0         0         0         0         0         0         0         0         0

Right hand side array of accelerations:

0.0000

0.4604

1.0232

-0.0921

0.4094

1.0234

0.9161

0.5895

2.2919

0.9161

0.6918

2.2919

1.9345

0.4628

1.0232

25.9147

0.6391

-27.9680

5.7175

7.7544

35.0903

-2.9693

-4.0429

6.8383

24.5885

96.1082

166.3353

-25.8226

-10.8585

Array of forces:

75.8947

208.0640

-56.9210

0

-9.8100

0

0

-9.8100

0

0

-9.8100

0

0

-19.6200

0

Mass Matrix:

2.0000         0         0         0         0         0         0         0         0         0         0         0         0         0         0

0    2.0000         0         0         0         0         0         0         0         0         0         0         0         0         0

0         0    0.1350         0         0         0         0         0         0         0         0         0         0         0         0

0         0         0    1.0000         0         0         0         0         0         0         0         0         0         0         0

0         0         0         0    1.0000         0         0         0         0         0         0         0         0         0         0

0         0         0         0         0    0.0560         0         0         0         0         0         0         0         0         0

0         0         0         0         0         0    1.0000         0         0         0         0         0         0         0         0

0         0         0         0         0         0         0    1.0000         0         0         0         0         0         0         0

0         0         0         0         0         0         0         0    0.0680         0         0         0         0         0         0

0         0         0         0         0         0         0         0         0    1.0000         0         0         0         0         0

0         0         0         0         0         0         0         0         0         0    1.0000         0         0         0         0

0         0         0         0         0         0         0         0         0         0         0    0.0570         0         0         0

0         0         0         0         0         0         0         0         0         0         0         0    2.0000         0         0

0         0         0         0         0         0         0         0         0         0         0         0         0    2.0000         0

0         0         0         0         0         0         0         0         0         0         0         0         0         0    0.0170

>>