# LabVIEW FRC 2012 Error Codes

Error 44055 FRC: The watchdog is disabled.

Error 44023 FRC: The edge type is disabled for the specified counter.

Error 44010 FRC: The specified sample rate is too fast.

Error -44001 FRC: The Axis camera username or password is invalid.

Error -44002 FRC: The host computer received an unknown version of image data from the CompactRIO device.

Error -44003 FRC: Operation failed due to a communication failure with the camera.

Error -44004 FRC: That camera address is already open. Try closing the address first.

Error -44005 FRC: Camera Get Image is being called, but no camera acquire loop is running. Try adding one of the camera aquire loop subVIs to the Vision Processing VI.

Error -44006 FRC: Custom I/O is not enabled or is not connected to the driver station.

Error -44007 FRC: The RefNum you are trying to Get does not exist in this RefNum Registry.

Error -44008 FRC: A Compressor VI is being called, but no Compressor Control Loop VI is running. Try adding the Compressor Control Loop subVI to the Periodic Tasks VI.

Error -44010 FRC: Use this VI on a host PC.

Error -44011 FRC: The analog channel is invalid.

Error -44012 FRC: The analog channel has already been allocated.

Error -44013 FRC: An invalid set operation occurred, and the analog channel was not allocated.

Error -44014 FRC: The scanList array input is invalid.

Error -44015 FRC: Invalid analog trigger index.

Error -44016 FRC: No analog trigger is available for use.

Error -44020 FRC: The counter is not set to a compatible mode.

Error -44021 FRC: <available>

Error -44022 FRC: No counter is available for use. Release an allocated counter before opening a new counter.

Error -44023 FRC: Invalid Counter Index Error.

Error -44024 FRC: Invalid Encoder Index Error.

Error -44025 FRC: An invalid set operation occurred, and the DIO resource was not allocated.

Error -44026 FRC: The DIO channel index is invalid.

Error -44027 FRC: The DIO channel has already been allocated.

Error -44030 FRC: The value of the DistancePerCount input is invalid. This input cannot have a value of 0.

Error -44031 FRC: <available>

Error -44032 FRC: No encoder is available for use. Release an allocated encoder before opening a new encoder.

Error -44033 FRC: The encoder index is invalid.

Error -44034 FRC: No encoder is available for use.

Error -44035 FRC: The PWM channel index is invalid.

Error -44036 FRC: An invalid set operation occurred, and the PWM channel was not allocated.

Error -44037 FRC: The PWM channel has already been allocated.

Error -44038 FRC: The loop that contains PWM SetValue is not running fast enough. This error can occur if the loop contains too much code, or if one or more other loops are starving the PWM SetValue loop.

Error -44040 FRC: The relay index is invalid.

Error -44041 FRC: The relay has already been allocated. Release the relay resource before allocating it again.

Error -44042 FRC: An invalid set operation occurred, and the relay channel was not allocated.

Error -44043 FRC: The loop that contains Relay Set is not running fast enough. This error can occur if the loop contains too much code, or if one or more other loops are starving the Relay Set loop.

Error -44045 FRC: The solenoid index is invalid.

Error -44046 FRC: The solenoid resource has already been allocated. Release the solenoid resource before allocating it again.

Error -44047 FRC: An invalid set operation occurred, and the solenoid channel was not allocated.

Error -44048 FRC: The loop that contains Solenoid Set is not running fast enough. This error can occur if the loop contains too much code, or if one or more other loops are starving the Solenoid Set loop.

Error -44050 FRC: OBSOLETE - Automatic mode for the ultrasonic sensor is enabled. The ultrasonic sensor can be pinged only if automatic mode is disabled.

Error -44055 FRC: No interrupt is available for use. Release an allocated interrupt before opening a new interrupt.

Error -44056 FRC: The interrupt index is invalid.

Error -44060 FRC: The HolonomicDrive VI works only with four-motor robots.

Error -44061 FRC: The loop that contains RobotDrive is not running fast enough. This error can occur if the loop contains too much code, or if one or more other loops are starving the RobotDrive loop.

Error -44065 FRC: The I2C sensor identification strings do not match.

Error -44070 FRC: The Analog Channel for the accumulator is invalid.

Error -44071 FRC: The analog channel is invalid.

Error -44072 FRC: The analog trigger channel is invalid.

Error -44073 FRC: The Analog Module for the accumulator is invalid.

Error -44075 FRC: The loop that contains MotorControl SetOutput is not running fast enough. This error can occur if the loop contains too much code, or if one or more other loops are starving the MotorControl SetOutput loop.

Error -44079 FRC: The CAN Jaguar firmware is not installed.

Error -44080 FRC: The DIO channel is invalid.

Error -44081 FRC: The PWM channel is invalid.

Error -44082 FRC: Invalid DIO Channel range.

Error -44083 FRC: The CAN Jaguar firmware is out of date.

Error -44084 FRC: The CAN Jaguar device number is out of range.

Error -44085 FRC: The FPGA bit file has an incorrect version or revision.

Error -44086 FRC: JaguarCANDriver detected insufficient buffer size.

Error -44087 FRC: JaguarCANDriver timed out waiting to receive a response from a Jaguar.

Error -44088 FRC: JaguarCANDriver prevented you from sending a message. The Message ID was identified as a restricted system message.

Error -44089 FRC: JaguarCANDriver has not been initialized. Please configure your cRIO to load a CAN Driver Plugin.

Error -44090 FRC: The Analog Module is invalid or not found. Make sure to set Analog Module to either Analog Module 1 or Analog Module 2 and ensure the module is plugged into the slot you specified.

Error -44091 FRC: The Digital Module is invalid or not found. Make sure to set Digital Module to either Digital Module 1 or Digital Module 2 and ensure the module is plugged into the slot you specified.

Error -44092 FRC: The Solenoid Module is invalid or not found. Make sure to set Solenoid Module to either Solenoid Module 1 or Solenoid Module 2 and ensure the module is plugged into the slot you specified.

Error -44095 FRC: Invalid I2C sensor ID.

Error -44097 FRC: DMA Engine is already started.

Error -44098 FRC: DMA Engine is not started.

Error -44099 FRC: DMA Engine is already in use.