// Definition of the data structure and routines for reading the sensor pack

#ifndef __sensors_h_

#define __sensors_h_

#define STX	(0x02)

typedef union

{

struct Data_tag

{

short				xPos;

short				yPos;

short				PosError;

unsigned char		DDCompas;

char				AngleError;

unsigned char		NullByte;

};

char chars[9];

} SensorPackType;

extern SensorPackType Sensors;

void SensorInitializePort (void);

void SensorPackRead (void);

#endif

#include "ifi_aliases.h"

#include "ifi_default.h"

#include "ifi_utilities.h"

#include "sensors.h"

#include "printf_lib.h"

// Added by Gene

#include <usart.h>

#include "delays.h"

SensorPackType Sensors;

/*******************************************************************************

* FUNCTION NAME: InitializeSensorPort

* PURPOSE:       Opens the serial port 2 for communicating with the scensors at

*                19.2k baud, 8 bits, no parity, one stop bit, and no flow control.

* CALLED FROM:   user_routines.c

* ARGUMENTS:     none

* RETURNS:       void

*******************************************************************************/

void SensorInitializePort (void)

{

Open2USART(USART_TX_INT_OFF &

USART_RX_INT_OFF &

USART_ASYNCH_MODE &

USART_EIGHT_BIT &

USART_CONT_RX &

USART_BRGH_HIGH,

128);

Delay1KTCYx( 50 ); /* Settling time */

}

/* Code to read Bills device. */

/*******************************************************************************

* FUNCTION NAME: ReadSensorPack

* PURPOSE:       Talks to the sensor pack to read the current data

* CALLED FROM:   user_routines.c

* ARGUMENTS:     none

* RETURNS:       void

*******************************************************************************/

void SensorPackRead (void)

{

int i, j;

RCSTA2bits.CREN = 1; // Re-enable the receiver

// Send an STX to the sensor pack to tell it to send the data

putc2USART(STX);

// Loop for the number of characters in the sensor pack data

for ( j = 0; j < 8/*sizeof(SensorPackType)*/; j++ )

{

// Wait a finite amount of time for the next character to arrive.

for ( i = 0; i < 20000; i++ )

if ( DataRdy2USART())

break;

// If a character arrived (we didn't timeout)

if ( i != 20000 )

{

// Grab the next charcter from the USART ant place it in our data structure

Sensors.chars[j] = getc2USART();

}

else

{

// If it failed then clear any possible errors

RCSTA2bits.CREN = 0; // Disable the receiver

RCSTA2bits.CREN = 1; // Re-enable the receiver

printf("Rx Fail DataRdy = %d\n",(int)DataRdy2USART());

}

}

RCSTA2bits.CREN = 0; // Disable the receiver

printf("x=%6d,y=%6d,pe=%6d,dd=%4d,ae=%4d\n",

Sensors.xPos,

Sensors.yPos,

Sensors.PosError,

(int)(unsigned char)Sensors.DDCompas,

(int)( char)Sensors.AngleError );

}