Project Name: Box with Lid

- Narrative

You and you partner are on a design team to design a jewelry box with a recessed lid. You will all be given the same size blank to start with; then your team will have to design an internal cavity to a specified size. The specified size will be a percentage of the total volume of the blank. You will be given only one piece of wax to make both lid and box.

- Concepts

Mass Properties		Area

Extrusion			Cylinder

Assembly Constraints	cu. in.

Fillets

Volume

(Instructor will tell your group which percentage to use)

- Geometric Requirements

- Stock Size:	1” x 2” x 3”

- Interior Volume Requirements:	_____46.6%			_____46.1

_____46.3%

_____46.5%			_____46.00

__x__46.2%

_____46.4%

- Interior corner radius:	.375” R

- Inquiry

- What is the volume of the box?

- What is the interior volume of the box?

- Determine the interior volume as a percentage of the total volume.

- Supplemental Information

- Rectangular Volume:	V = W x L x H

- Cylindrical Volume:	V =  R2

Part 2:	Box Lid

- Narrative

Your team will now design a lid to fit your box.

- Geometric Requirements

- Stock Size:	.5” x 2” x  3”

- Clearance:	.03125” between box lid and sides of box.

- Lip of box will project .125” into box.

- Drawing Requirements

- Inventor drawing of the design.  Both box and lid.  Dimensioned multi-view drawings with an Isometric.

- Master cam verification of both.  Print screen on word.

- WLPM1000 Program verified and a copy of the described code for each.

- Description of the project and procedures to create the box.

- Analysis of the project.

- Actual production of the box. Photos of completed box