GENERAL OVERVIEW

In order to be considered a Vex Sumobot, it shall contain at least 
the following: 
- One Vex CPU minimum (whether used or not - other processors are also 
allowed) 
- ONLY uses Vex Motors and/or Vex Servos for drivetrain motion 
- ONLY uses at most one VEX (or "VEX Equivalent") NiCd battery 
       for all power needs. 
- Meets weight, initial size, and drivetrain motor usage restrictions 
       for its class.

In addition: 
- Additional Motors, solonoids and/or motor controls - ALLOWED, for 
      NON-drivetrain purposes. 
- Sensors - UNLIMITED - any type, brand, signal conditioning, and 
coprocessing 
      is allowed, as long as it does NOT pose a physical danger to 
      other robots, participants, and audience, nor interfere with 
      activity in another Dohyo.  (No CO2 lasers, "flame illuminators", 
etc.) 
- Sensor Spoofing - ALLOWED, as long as it does NOT interfere with 
      adjacent Doyhos, the pit area, nor pose any risk to robots or 
attendees. 
      Robots in the Pits or in line awaiting competition may NOT 
interfere with 
      Dohyo action at ANY time.

SUMOBOT CONSTRUCTION CLASSES

There will be THREE (3) construction classes:  Wheelbot, Omnibot, and 
Walkerbot 
All robots will compete together, though different construction rules 
apply 
to each class.

A WALKERBOT is defined as any robot that can ONLY move via "reciprocal 
motion of limbs" of some kind.  It must NOT be capable of motion via 
any kind or roller, wheel, or tire.

An OMNIBOT is defined as any machine that ONLY uses omniwheels and/or 
mecanum wheels for drivetrain motion (does not contain ANY standard 
wheels).

A WHEELBOT is defined as any machine capable of rolling drivetrain 
motion in any way shape or form incorporating at least one non-omni 
or mecanum drive wheel in its design, whether or not such wheel is ever 
used.

Example:  A machine that can move by either standard wheels, omniwheels 
or limbs shall be considered a Wheelbot for construction rules 
purposes, 
and be limited by the Wheelbot specs.

WEIGHT MAXIMUM

- Wheelbot  = 6 lbs max 
- Omnibot   = 7 lbs max 
- Walkerbot = 9 lbs max

STARTING SIZE

- Wheelbot  = 12" x 12" initial footprint Maximum. 
- Omnibot   = 12" x 12" initial footprint Maximum. 
- Walkerbot = 14" x 14" initial footprint Maximum.

Robots may deploy from that size upon startup. 
There is NO initial height limit on any class.

MOTOR USAGE

All Vex Sumobots shall ONLY use Vex branded motors or servos for 
motion. 
The motors may NOT be modified (no rewinding allowed), but the 
geartrains 
and cases MAY be modified, including material changes.  Nontraction 
motors 
or solonoids may be of any type, but must be powered off of the single 
Vex class NiCd pack.

Comment: WARNING - If clutches are removed, the plastic Vex gears may 
shear 
during competition.  No gear replacement will be allowed during a 
match.

There is no total maximum number of Vex motors in a bot, as long 
as it remains within the weight limit, but DRIVETRAINS shall be 
limited in motor usage as follows:

- Wheelbots - shall not exceed FOUR Vex drivetrain motors active at one 
time.

- Omnibots  - shall not exceed SIX Vex drivetrain motors active at one 
time.

- Walkerbots - UNLIMITED motors for drivetrain usage at any time.

There is no limit to the TOTAL number of Vex motors on a Vex Sumobot 
(up to the weight maximum),but the remaining motors can not be used 
for propulsion in any way.

AUTOMOMOUS OPERATION

Sumobots shall be Autonomously operated, by the Vex MPU.  Additional 
processors beyond the Vex MPU may be used for any purpose.

Sumobots shall have easily accessible Start and large Disable buttons, 
For ease of inspection, it is suggested that the Disable button be a 
pull on / push off mushroom cap button, wired in series with the 
battery lead, 
A software controlled Disable button may be allowed, with prior code 
inspection.

Robots may begin scanning for opponents before Start is operated, 
but ALL other motions (drivetrain, shape changing, and payload) 
shall be supressed until FIVE (5) seconds after Start is hit.

If the Disable button is hit, the sumobot must IMMEDIATELY freeze, 
and RELAX *all* moving devices. Upon powerdown, ALL grippers and 
capture 
devices must either automatically "relax", or simply and intuitively 
"release" whatever they hold without powerup again via a manual 
catch or pin.

For safety: 
- NO finger pinch hazards shall exist. 
- All pinch forces exerted shall not exceed 20 lbs at any time, 
- Any created "ratcheting devices" must be proven to be human finger 
safe, 
  and easily released without power applied, or they shall not be 
allowed. 
- Any "Venus Flytrap" / "Mousetrap" (sensor or mechanically driven 
gripper 
  closing) devices shall not be able to harm fingers by accidental 
actuation.

Examples: 
- Creating a J-hook or a closing ring that can only hook or capture an 
  opponent but not pinch is much preferred over a tighly closing "fist 
  gripper" that could potentially pinch or cut fingers. 
- A Venus Flytrap device might use lightweight limited force springs 
  instead of motors, to keep it from becoming a surprise finger hazard.

TIRES, TREADS, FEET & TRACTION

There are NO restrictions on tire/tread/foot design, weight, size, 
shape, 
type, or application, as long as the entire robot meets both weight and 
initial dimension requirements for its class, AND it does not adhere 
to, 
contaminate, or damage the Dohyo surface in any way.  Vex tires MAY be 
modified to increase their effectiveness. Custom tires, treads and 
footpads 
are allowed.

Sumobots may not attempt to adhere to the Dohyo surface beyond simple 
traction. 
NO glues, oil slicks, water, screws, studs, nails, residues, adhesives,

or vacuum, shall be used for "traction enhancement or modification" of 
themselves, or opponents at ANY time.  Robots that deliberately modify, 
damage, 
or contaminate the Dohyo surface in any way are immediately 
disqualified, 
and at the judge's discretion may be banned from further competition.

ENTAGLEMENT - PROHIBITED

You may not use probes, cords, nets, tethers, or other devices to 
interfere 
with the normal operation of opponent machines via entaglement or 
jamming. 
Examples: Casting a net over an opponent to snare an opponent, then 
using a crane to lift and dump them out of the Dohyo is prohibited. 
Inserting a stick into an opponent's mechanism to jam it is prohibited,

though siezing it with a gripper while attempting a wrestling hold 
IS allowed.  You may not run separate sub-bots via a tether, as the 
tether would constitute an entaglement hazard.

SEPARATION - ALLOWED

Robots MAY drop subsections of itself to confuse opponents, but 
entanglement 
is prohibited, so no "tethers" will be allowed.  Example: dropping a 
dummy cone 
is permitted. NOTE - This MAY be a risky strategy, because if ANY part 
of 
one robot (even a screw or "decoy cone") is ejected from the Dohyo by 
accident or an opponent's actions, the ENTIRE robot is considered "out"

for scoring purposes.

OPPONENT DAMAGE, DOHYO CONTAMINATION, AND HAZARDOUS MATERIALS ARE 
PROHIBITED

NO direct damage, threat, or hazard to opponents, participants, field 
personnel, or audience shall be allowed in any way, shape, or form at 
any time.

Grippers may not deliberately crush opponent nor pose a pinch hazard 
for humans.  Grippers must be able to relax and release when the robot 
is disabled.

Examples: 
- No "BattleBot" [tm] style devices are allowed (saws, hammers, etc.) 
  that phyically damage other robots. 
- High power laser rangefinders, UV emitters, or strobelights that 
  may pose a vision hazard to spectators are prohibited. 
- No projectiles shall be ejected from any robot at any time. 
- No fluids, nor dangerous substances shall be used in any form in 
robot 
  construction or operation (mercury, fuels, oils, breakable glass, 
sharp 
  edges, etc.). 
- All EMP devices, flames, wire cutters, electric shock, opponent 
shorting, 
  or other methods of "disablement via systemic damage" are prohibited.

Comments:

1) This is WRESTLING, not BattleBots [TM].  We wish you to SAFELY 
eject your opponent from the Dohyo, without permanent damage to the 
opponent, Dohyo, nor risk to all others attending.  Siezing and lifting 
opponent robots to THROW them out IS allowed (and encouraged), but 
destruction of them is NOT appropriate.

2) We wish to HIGHLY encourage "payload" development.  Unlike many 
other 
Sumobot contests, we do NOT wish this contest devolving into simple 
"drivtrain only robots" that only use their entire weight allowance to 
stack as many drive motors onto a bare chassis as possible to bully 
other robots out  To that end, we've restricted the motor type and 
number used for DRIVETRAIN purposes, but are NOT restricting OTHER 
motors, 
nor their controls.  We hope that the enhanced weight allowances over 
similar Sumobot contests will encourage the emergence of creative 
gripper, 
flipper, forklift, and other payload designs to sieze, lift, throw, 
carry, 
or "flip & shovel" opponents out of the Dohyo instead of simply pushing

them out.

3) Omnibots are less efficient than Wheelbots, so they are allowed 
more drive motors.

4) Walkerbots are at a significant speed and strategy disadvantage, so 
in compensation they have been given both much larger size and weight 
limits and unlimited motors.

5) Tires were made "unlimited" to encourage creativity in traction 
and overall design.  Our testing leads us to believe that stock Vex 
tires will still produce very competitive robots.

6) We wish this to be a VEX Sumobot contest, so there is a bare 
minimum of a Vex CPU, motors, and batteries (or their equivalent) 
required.  However, we highly encourage coprocessors, advanced 
sensors, sensor spoofing, and payload device development.  In addition, 
the chassis and tires/treads/feet may include non-Vex materials.

Our intent is that these rules should allow a purely Vex machine 
created from only a Vex Starter Kit, a Programming Kit, a Vex Battery 
Pack, and a few hardware store items to be very competitive.  Yet, 
we hope there's still enough flexibility and weight/size limits 
for advanced teams with unique talents to attempt novel sensor 
and payload designs.

Good luck!