DashBoard

By Lohit Yerva

Team 226 “Hammerheads”

------------------------------------------------------------------------------------------------------------

July 31, 2007 – Original Code

Aug 24, 2007 – PWM text memory feature (V 0.2)

System variables (ex. Main battery voltage) added DataView (V 0.2)

------------------------------------------------------------------------------------------------------------

I knew that no matter how much I begged and screamed, the robot would never be completed with enough time for any decent programming time left. I knew many were on the same boat. Thus, I embarked on developing a program that would allow for more to be accomplished in less time. You never have enough time for programming! Debugging was the most time consuming part of the software development. Since the IFI hardware restrictions don’t allow for the use of In-circuit debuggers, I realized that viewing variables values as they progressed through a function was the best way to debug. This application is intended to improve debugging efficiency by reducing the number of code downloads, allowing to view 10 different variables simultaneously and graph 5 of them, view tons of camera data, and directly control PWMs for hardware diagnostics, all at the same time and in real time. No re-flashing is required to change which variable is being viewed in each column!

This program has been in development for over a year. The DataView has been field tested during the 2007 season and proven to be very effective. It has been the most complex project I have ever worked on with a combination of RC hardware and software along with Visual C++ application development. I have tried to maintain efficiency and reliability in the RC code. But on the Visual C++ application side I only worried about reliability since the PC processor is far more capable and thus, easily able to process application code.

CAUTION: I have tried my best to put the highest priority on safety. The PWM control has not been field tested, thus I cannot guarantee %100 safety while using the PWM. Yet the system has many fail-safes.  If the program cord is disconnected, serial port turned off, or the RESET/STOP is clicked, the pwms, relays, and joysticks controls will all reset to default (127 or 0).

The application was designed to be intuitive and straightforward.

How to use the DashBoard:

ReStart:

If you had to reset the RC for any reason, hit the “main” ReStart next the close serial port check box to get everything working like before. The ReStart in the DataView or Camera will just restart that particular item.

DataView:

In the “Variable Type”:

PWM = pwms

EXtra = extra variables( look under extra features)

JStick = Operator Interface analog inputs

JButton = Operator Interface digital inputs

Analog = Analog inputs

D In = digital inputs

D Out = digital outputs

Relays = Relay outputs

Apwm = Abstract pwms(look under extra features)

In the “Variable Number”:

You have the choices under each category: ex. 1-16 for the PWMs

Camera:

There isn’t anything other than disable check box which just freezes everything in camera view.

PWM:

The check boxes in front of each pwm let you choose where you want to control that pwm in the RC or not. “DeSelect/Select ALL” button checks or un-checks all pwms.  The “CEN” in front of each pwm will center that pwm to 127. The RESET button will enable control for all pwms( check all boxes) and center all pwms. This was designed for safety. It will take control of all the pwms and assign them all 127.

FEATURES:

Real-time Graphing:

Dynamic scaling

EXtra:

There are 16 extra “int” variables that can be assigned any values and view on the

DashBoard. In the code just put e_0X or x_0X = variable, X being from 1-8 and you can view the values under the corresponding name in the EXtra category. It can be up to an ‘int’ variable(-32767 to 32767).

Ab PWM:

The “regular” PWMs are the ones that go directly out the pwm output pins on RC. But I know some use the -127 to 127 range or some type of abstraction layer. This feature is for those people.

Camera:

If you are using the “bells and whistles” version of the camera code, don’t worry, the interactive terminal based menus will still work. You just have to disable the serial port or close the DashBoard to be able to open the terminal window. Likewise, if you want to use the DashBoard you have to close the terminal window. Only one program can control the serial port at any given time.

PWM names memory:

Now the text/name typed into each textbox on top of the individual PWM will be stored in a text file for the next load.

System Variables:

MainBat = Main Battery Voltage

BackBat = Backup Battery Voltage

UsMoBy = User Mode Byte

UbyteX = User Bytes 1-6

LEDb2 = LED Byte 2( Used for the 3 Switch lights)

PackNum = Current Packet Number

NOTE: The reason for its current release is to see if people are interested, and for me to understand problems that others might have with their RC or PC while implementing this setup. Right now only the DataView (without data logging), Camera, and PWM work. If people are interested and not too many serious problems are experienced, then a more complete package will be released later.

If you want to test it out first, I have included a compiled hex file for the 06-7 RC with the source code. Download this to the RC using the IFI loader and fire up the DashBoard for testing. You still need to follow instructions 1 and 2 from “How to get started.”

CAUTION: : I have tried my best to put the highest priority on safety. The PWM control has not been field tested, thus I cannot guarantee %100 safety while using the PWM. Yet the system has many fail-safes.  If the program cord is disconnected, serial port turned off, or the RESET/STOP is clicked, the pwms, relays, and joysticks controls will all reset to default (127 or 0).

I need to give a special thanks to Kevin Watson for all his marvelous code.

Requirements:

PC:

.NET Framework 2 or above

MPLAB

RC:

Kevin Watson’s Serial Port

Kevin Watson’s Camera Code (if using the camera feature)

Up the RX1 buffer to 64 in serial_ports.h

Still under development:

Joystick (Allows you to simulate all of the OI inputs from the PC)

DataView data Logging into text file

How to get it working:

- Install .NET Framework 2 to run the application, if already not installed.

(In XP or 2000): Control Panel | Add and Remove Programs

(In Vista): Control Panel | Programs

In both check if you have installed “Microsoft .NET Framework 2.0”

Download it here:

http://www.microsoft.com/downloads/details.aspx?FamilyID=0856EACB-4362-4B0D-8EDD-AAB15C5E04F5&displaylang=en

2) Disable the dial-up/fax modems on your computer in the device manager. For   some reason the serial port won’t work correctly with them on.

(In XP) Right-Click My Computer | Properties | Hardware | Device Manager | Modems

(In Vista) Start | Control Panel | System and Maintenance | System | Device Manager | Modems

- Now you will need MPLAB with C18 V2.4 installed, if you don’t already, to edit your RC code to build the new additions.

MPLAB: http://www.microchip.com/stellent/idcplg?IdcService=SS_GET_PAGE&nodeId=1406&dDocName=en019469

C18 V2.4:

Sorry, you are on your own. No legal way to get it.

- You need at least Kevin Watson’s serial port code to get this running on the RC. You can get it form here: http://kevin.org/frc/ .

- IMPORTANT: In serial_ports.h up the RX_1_QUEUE_SIZE from 32 to 64

- Now you need to add the DashBoard.c, DashBoard.h, and DashBoard_readme.txt files into the build.

First, copy these files into the folder where your code resides.

IN MPLAB:

Right-Click on Source Files and Header Files | Add files

Now add the #include “DashBoard.h” at the top the user_routines.c and any other .c file that you will be calling the extra variables for use in.

7)  After adding the files, IN DashBoard.h:

If you are using his camera code:

Uncomment #define CAMERA

If you are using the “bells and whistles” version of the camera code:

Uncomment #define BELLS_AND_WHISTLES_CAMERA

If you are using his ADC code:

Uncomment #define KEVIN_ANALOG

8)  I have included a user_routines.c file to help people setup theirs.

---The easiest way to do it, if you ARE using the “bells and whistles” version of the Camera code:

1) Create 2 “unsigned char” global variables:

unsigned char camera_menu_active = 0;

unsigned char tracking_menu_active = 0;

2) Copy both the “SerialPort_One_Access” and “Camera” functions into your user_routines.c file.

3)  Uncomment the “#define BELLS_AND_WHISTLES_CAMERA” and “#define CAMERA” in the DashBoard.h

4) And, finally, setup your “Process_Data_From_Master_uP” function exactly like the way it is in the included user_routines.c file.

---If you are NOT using the “bells and whistles version” of the Camera code:

- Just copy the “SerialPort_One_Access” function into your from the user_routines.c file

- Comment out the “#define BELLS_AND_WHISTLES_CAMERA” in the DashBoard.h

4) Then setup your “Process_Data_From_Master_uP” function exactly like the way it is in the included user_routines.c file. EXECPT replace the Camera() function with Camera_Handler() and Servo_Track() functions. Place this right after the SerialPort_One_Access() function:

if(Link_Missed_Count == 0)

Tracking_Info_Terminal();

---If you are NOT using the Camera code at all:

- Just copy the “SerialPort_One_Access” function into your user_routines.c file

- Comment out the “#define BELLS_AND_WHISTLES_CAMERA” and “#define CAMERA” in the DashBoard.h

3) Then setup your “Process_Data_From_Master_uP” function exactly like the way it is in the included user_routines.c file. EXECPT no Camera() function

9) Compile and download. Ideally it should start working.

CHECK:

RC is connected to the OI via tether or radios

RC is connect to the computer via Program port