90% Throttle Response - Combined

-100.00%

-80.00%

-60.00%

-40.00%

-20.00%

0.00%

20.00%

40.00%

60.00%

80.00%

100.00%

1

6

11

16

21

26

31

36

41

46

51

56

61

66

71

76

81

Series1

Series2

Series3

This graph is left motor (series 2) and right motor (series 3) versus joystick input (series 1).  This is a section of the previous motor response data with data from the both wheels.  We can see that the two curves are similarly non-linear.  We also can clearly see that the wheels are offset from each other – most notably at the lower rpm values.

The program results in a table which is placed into program (read-only) memory.  A joystick value is used as an index value and a compensated PWM value for that joystick input is accessed from the table and fed to the motors.  There is a separate right versus left table to speed match the motors to the desired value.

Here we see the linear joystick input as “Ndx” ranging from 127 to 255.  The “RgtNdx” is the output of the lookup table for “Ndx”.  The “Target” is the specified/desired response curve – in this case a linear response was specified.  The “Rgt-V” is the measured right wheel velocity.  As can be seen there is a significant stair step at low speeds.  The stair steps are due to the digital nature of the design.  The stair steps are different for left and right wheels.   These digital stair steps within the curve diminish in breadth as we get above 90% speeds.

There are several other things to note about the graph.  First it is easy to see the dead band built into the RgtNdx values.  The victor motor controls have a built in dead band of +/- 7.  The above table mimics the same range.  To increase the dead band, the lookup table would have more motor idle values placed in more locations.