Subject: Acoustic Measurements at FIRST events

This document outlines the requirements for acoustic measurements at FIRST events and explains the rational behind the requirements.

Acoustic measurements need to be performed at the defined locations each hour during the event.  This will gather the necessary information to solidify or negate the concern of noise levels at FIRST events.

The following information about the measuring device will be needed.  The name of the user, Team number, Manufacturer, Model Number, Serial Number, Calibration Date and when the next Calibration is due.  This information will have to be filled in on the spreadsheet where the data is being recorded.

Sound level readings should be taken in multiple locations over the course of each day of the event.  In an effort to balance recording these readings and be a part of your team, three reading locations were chosen.  We need people to record at 3 locations every hour during the events.  They should take 2 readings at each station, if the meter is able to do this.  We would like both the average (dBA) and the peak (dB) readings.  The 3 locations chosen to take measurements from are all in the stands facing the field.  They are on each alliance base line and at the centerline of the field, 10 rows up in the stands.  The meter needs to be held at a height of 44 inches and you need to take approximately 15 seconds for the meter to settle out at each location to take the reading.  The locations are defined below.

Alliance line

Typically Red

Alliance line

Typically Blue

Field Center Line

Measurement locations

in stands

10 rows back

10 rows back

The measurement location in the stands was chosen because it is accessible by everyone on any team where a field location is limited to those who are part of the competition team or have media passes.  Competition team members are restricted as to when they can be in the field area and therefore cannot be used to gather this data.  Media passes are not designated for this purpose and therefore we will not use them in this manner.

Table I below is a representation of the sheet you will be filling out at each event. The dBA and dB levels are different selections on the meters.  Please select the correct scale for the reading you want to take.  If you have only one selection on your meter, record this data and leave the other column blank noting the meter limitations on the spreadsheet.  The complete excel spreadsheet will be made available to be filled out during the events.

Table I

| Suggested Recording | Actual Time | Blue Alliance Base line |  |
| --- | --- | --- | --- |
| Time | of Readings | dBA | dB |
| 9:00 AM |  |  |  |
| 10:00 AM |  |  |  |
| 11:00 AM |  |  |  |
| 12:00 PM |  |  |  |
| 1:00 PM |  |  |  |
| 2:00 PM |  |  |  |
| 3:00 PM |  |  |  |
| 4:00 PM |  |  |  |
| 5:00 PM |  |  |  |

The 44-inch height for the meter reading correlates to the height of the ears of a sitting person at the 90th percentile of the human population of a young adult of approximately 17 years of age and male.  The seat considered for this equation is that of the average seat installed in arenas.  Seat height varies over the years and by manufacturer, therefore the average seat height installed in arenas that are between 10 and 20 years old was chosen.

From NEMA Standards Publication No. MS4-1989, page 3, paragraph 1.6.2:

The OSHA limit for Lpeak (unweighted) is 200 pascals (or 140 dB referenced to 20 micropascals). The OSHA limit for Leq (A-weighted) for occupationally exposed individuals is 90 dBA averaged over 8 hours per 24 hour day (or 105 dBA over 1 hour). The OSHA Leq standard is intended for occupational exposure.

Table II

| Leq Sound Pressure Level dBA | Permitted Time, Hours |
| --- | --- |
| less than 80 | no limit |
| 80 | 32 |
| 85 | 16 |
| 90 | 8 |
| 95 | 4 |
| 100 | 2 |
| 105 | 1 |
| 110 | ½ |
| 115 | ¼ |
| Above 115 | Not permitted |

Table II: Weighting Factors for OSHA (table 16a of OSHA 1910.95)

Table II lists the OSHA Leq sound pressure levels and the associated time permitted at these levels.

When you have completed the spreadsheet for the competition you were at please e-mail the spreadsheet to me at megasparks2062@yahoo.com

Lets all work together to change FIRST to make it better for everyone.

Bryan Sowatzke

Mentor Team 2062