## DS Dashboard Data Format

The DS dashboard data uses a simple data format. By analyzing the LabView dashboard sample and analyzing network traffic I was able to write this documentation.

The DS sends UDP packets from port 1170 to 10.xx.yy.6 port 1165. The data packet is 1018 bytes in length. The first 26 bytes are DS, field, and robot status data. The next 984 bytes are user configurable. The remaining 8 bytes, well I have no idea what they are. They could be a checksum, but I haven’t confirmed this.

The data contained in the DS status data is:

| Data | Type/notes |
| --- | --- |
| Packet Number | UInt16 |
| DS Digital Inputs | 1 Byte, each bit is an intput |
| DS Digital Outputs | 1 Byte, each bit is an output |
| Battery level | 2 bytes, I haven’t tried to convert them to a voltage, but they may just be scaled. |
| Status Byte | Byte, each bit represents a piece of status info. I haven’t finished the documentation on this. |
| Error Byte | Byte, each bit represents an error state. I haven’t finished the documentation on this. |
| Team Number | 2 bytes, team number is calculated using the following formula: TeamNumber = Byte1 * 100 + Byte2 |
| Version Month/Day/Year/Rev | 2 Bytes each(8 bytes) haven’t finished docs. |
| UnusedBuffer1 | UInt32 |
| UnusedBuffer2 | UInt16 |
| ReplyPacketNumber | UInt16 |
| User Data | 984 bytes |
| Unknown | 8 bytes |

## Default data packer user data format

| Data | Type/notes |
| --- | --- |
| Sequence Number | 1 byte |
| Length of Printed String | 4 bytes |
| Printed string | Length indicated in the previous four bytes |
| Length of Error String | 4 bytes |
| Error String | Length indicated in the previous four bytes |
| Length of user data | 4 bytes |
| User bytes | Length indicated in the previous four bytes |

## Dashboard camera interface

The camera communication takes place on TCP port 1180. Since this communication is user initiated the PC IP address and port numbers are not relevant.

No data needs to be sent from the dashboard PC. Simply making a TCP connection on the correct port is sufficient to initiate image transfers at a regular interval.

| Date | Type/Notes |
| --- | --- |
| Header | 4bytes, {1, 0, 0, 0} |
| Length of camera data | UInt32 |
| ImageData | Bytes, length indicated in the previous four bytes. Jpeg encoded |