'{$STAMP BS2sx}

'========== DECLARE VARIABLES ================================================

'=============================================================================

'  Below is a list of declared input and output variables.  Comment or un-comment

'  the variables as needed.  Declare any additional variables required in

'  your main program loop.  Note that you may only use 26 total variables.

'---------- Operator Interface (OI) - Analog Inputs --------------------------

p1_y		VAR byte	'Port 1, Y-axis on Joystick

p1_x		VAR byte	'Port 1, X-axis on Joystick

p2_y		VAR byte	'Port 2, Y-axis on Joystick

p2_x		VAR byte	'Port 2, X-axis on Joystick

'---------- Misc. ------------------------------------------------------------

PB_mode	VAR byte

'---------- Aliases for the Pbasic Mode Byte (PB_mode) -----------------------

'  The last bit of the PB_mode byte (aliased as comp_mode) indicates the status

'  of the Competition Control, either Enabled or Disabled.  This indicates the

'  starting and stopping of rounds at the competitions.  Comp_mode is the same

'  as the "Disabled" LED on the Operator Interface.

'  Comp_mode = 1 for Enabled, 0 for Disabled.

comp_mode		VAR PB_mode.bit7

'========= DEFINE CONSTANTS FOR INITIALIZATION ===============================

'=============================================================================

'  The initialization code is used to select the input data used by PBASIC.

'  The Master micro-processor (uP) sends the data you select to the BS2SX

'  PBASIC uP.  You may select up to 26 constants, corresponding

'  to 26 variables, from the 32 available to you.  Make sure that you have

'  variables for all the bytes recieved in the serin command.

'

'  The constants below have a "c_" prefix, as compared to the variables that

'  they will represent.

'

'  Set the Constants below to 1 for each data byte you want to recieve.

'  Set the Constants below to 0 for the unneeded data bytes.

'---------- Set the Initialization constants you want to read ----------------

c_p1_y		CON	1

c_p2_y		CON	1

c_p3_y		CON	0

c_p4_y		CON	0

c_p1_x		CON	1

c_p2_x		CON	1

c_p3_x		CON	0

c_p4_x		CON	0

c_p1_wheel	CON	0

c_p2_wheel	CON	0

c_p3_wheel	CON	0

c_p4_wheel	CON	0

c_p1_aux	CON	0

c_p2_aux	CON	0

c_p3_aux	CON	0

c_p4_aux	CON	0

c_oi_swA	CON	0

c_oi_swB	CON	0

c_sensor1	CON	0

c_sensor2	CON	0

c_sensor3	CON	0

c_sensor4	CON	0

c_sensor5	CON	0

c_sensor6	CON	0

c_sensor7	CON	0

c_batt_volt	CON	0

c_rc_swA	CON	0

c_rc_swB	CON	0

c_delta_t	CON	0

c_PB_mode	CON	1

c_packet_num	CON	0

c_res01		CON	0

'---------- Initialization Constant VOLTAGE - USER DEFINED -------------------

'  This is the 'Low Battery' detect voltage.  The 'Low Battery' LED will

'  blink when the voltage drops below this value.

'  Basically set VOLTAGE = INT ((DESIRED FLASH VOLTAGE + 0.4) * 16.3)

'  Example, for a 10 Volt trigger, set Voltage eq 170.

dataInitVolt	CON 153	'9.0 Volts

'========== DEFINE CONSTANTS (DO NOT CHANGE) =================================

'=============================================================================

' Baud rate for communications with User CPU

OUTBAUD	CON	20	' (62500, 8N1, Noninverted)

INBAUD	CON	20	' (62500, 8N1, Noninverted)

USERCPU	CON 4

FPIN		CON 1

COMA		CON 1

COMB		CON 2

COMC		CON 3

'========== MAIN PROGRAM =====================================================

'=============================================================================

'---------- Input & Output Declarations --------------------------------------

Output	COMB

Input		COMA

Input		COMC

Output	7	'define Basic Run LED on RC => out7

Output      8	'define Robot Feedback LED => out8	=> PWM1 Green

Output      9	'define Robot Feedback LED => out9	=> PWM1 Red

Output      10	'define Robot Feedback LED => out10	=> PWM2 Green

Output      11	'define Robot Feedback LED => out11 => PWM2 Red

Output      12	'define Robot Feedback LED => out12 => Relay1 Red

Output      13	'define Robot Feedback LED => out13 => Relay1 Green

Output      14	'define Robot Feedback LED => out14 => Relay2 Red

Output      15	'define Robot Feedback LED => out15 => Relay2 Green

'---------- Initialize Inputs & Outputs --------------------------------------

p1_y = 127		'Port 1, Y-axis on Joystick

p2_y = 127		'Port 2, Y-axis on Joystick

'========== PBASIC - MASTER uP INITIALIZATION ROUTINE ========================

'=============================================================================

'  DO NOT CHANGE THIS!  DO NOT MOVE THIS!

'  The init routine sends 5 bytes to the Master uP, defining which data bytes to receive.

'  1)  Collect init.

'  2)  Lower the COMA line, which is the clk line for the shift out command.

'  3)  Lower COMB line to tell pic that we are ready to send init data.

'  4)  Wait for pic to lower the COMC line, signaling pic is ready for data.

'  5)  Now send out init dat to pic, all 5 bytes.

'  6)  Now set direction and levels for the COMA and COMB pins.

tempA		CON	c_p3_x <<1 + c_p4_x <<1 + c_p1_x <<1 + c_p2_x <<1 + c_rc_swB

dataInitA	CON	tempA <<1 + c_rc_swA <<1 + c_oi_swB <<1 + c_oi_swA

tempB		CON	c_sensor4 <<1 + c_sensor3 <<1 + c_p1_y <<1 + c_p2_y <<1 + c_sensor2

dataInitB	CON	tempB <<1 + c_sensor1 <<1 + c_packet_num <<1 + c_PB_mode

tempC		CON	c_batt_volt <<1 + c_sensor7 <<1 + c_p1_wheel <<1 + c_p2_wheel <<1 + c_sensor6

dataInitC	CON	tempC <<1 + c_sensor5 <<1 + c_p3_y <<1 + c_p4_y

tempD		CON	c_res01 <<1 + c_delta_t <<1 + c_p3_aux <<1 + c_p4_aux <<1 + c_p1_aux

dataInitD	CON	tempD <<1 + c_p2_aux  <<1 + c_p3_wheel <<1 + c_p4_wheel

Output	COMA

low		COMA

low		COMB

Wait_init:	if IN3 = 1 then Wait_init:

Shiftout	COMB,COMA,1, [dataInitA,dataInitB,dataInitC,dataInitD,dataInitVolt]

Input		COMA

high		COMB

'========== MAIN LOOP ========================================================

'=============================================================================

'***********************

'Make new variables

'***********************

L_PWM		VAR byte	'Left PWM output variable (1 byte, 0->255)

R_PWM		VAR byte	'Right PWM output variable (1 byte, 0->255)

'***************************************

'Initialize left and right PWM variables

'***************************************

L_PWM = 127

R_PWM = 127

MainLoop:

'---------- Serin Command - Get Data from Master uP --------------------------

'  Construct the "serin" command using the following rules:

'  1) There must be one variable for every input defined in the "Define Constants for Init" section.

'  2) The order must match the order in the EXAMPLE SERIN COMMAND below.

'  3) The total number of all variables may not exceed 26.

'  4) Only use one "Serin" command.

'  5) The Serin command must occupy one line.

'

'  If you see a BASIC INIT ERR on the Robot Controller after programming and pressing RESET, then

'  there is a problem with the Serin command below.  Check the number of variables.  A BASIC INIT ERR

'  will not occur if you have the variables in the wrong order, however your code will not work correctly.

'

'  EXAMPLE SERIN COMMAND

'  This example exceed the 26 variable limit and is not on one line:

'

'  Serin COMA\COMB, INBAUD, [oi_swA,oi_swB,rc_swA,rc_swB,p2_x,p1_x,p4_x,p3_x,PB_mode,packet_num,sensor1,

'				     sensor2,p2_y,p1_y,sensor3,sensor4,p4_y,p3_y,sensor5,sensor6,p2_wheel,p1_wheel,

'				     sensor7,sensor8,p4_wheel,p3_wheel,p2_aux,p1_aux,p4_aux,p3_aux,delta_t,res01]

'

'*******************************************

'Read in Joystick values

'*******************************************

Serin COMA\COMB, INBAUD, [p2_x,p1_x,PB_mode,p2_y,p1_y]

'========== PERFORM OPERATIONS ===============================================

'  Add your custom code here.

'  Delete any of the following sections below (except for Output Data) as desired.

'*******************************************

'Calculate output PWM values

'*******************************************

L_PWM = p1_y - p1_x + 127		'Calculate left PWM value to be send to left motor

R_PWM = p1_y + p1_x - 127		'Calculate right PWM value to be send to right motor

'========== OUTPUT DATA ======================================================

'=============================================================================

'  The Serout line sends data to the Output uP.  The Output uP passes this to each PWM 1-16

'  and Relay 1-8.  The Output uP will not output data if there is no communication with the

'  Operator Interface or if the Competition Mode is Disabled.  Do not delete any elements

'  from the Serout array.  Set unused PWM outputs to 127.  Set unused relay outputs to 0.

'

'  Serout USERCPU, OUTBAUD, [255,255,(PWM1),relayA,(PWM2),relayB,(PWM3),(PWM4),(PWM5),(PWM6),(PWM7),(PWM8),(PWM9),(PWM10),(PWM11),(PWM12),(PWM13),(PWM14),(PWM15),(PWM16)]

'*******************************************

'Send PWM values to motors

'*******************************************

Serout USERCPU, OUTBAUD, [255,255,p1_y,0,p1_y,0,p2_y,p2_y,127,127,127,127,127,127,127,127,127,127,127,127]

Goto MainLoop:

Stop