Team 537 Dashboard Protocol Analysis

Many thanks to Team 1103 and their assistance and information that helped create this document.

The DS sends UDP packets from port 1170 to 10.xx.yy.6 port 1165. The data packet is 1018 bytes in length. The first 26 bytes are DS, field, and robot status data. The next 984 bytes are user configurable.  The final eight bytes consist of four bytes of padding followed by a crc32 checksum.

| Hex Memory Address | Assumed Data | Type/Additional Info |
| --- | --- | --- |
| Driver Station Status Data: |  |  |
| 00-01 | Packet Number | 16-bit integer (assumed to be unsigned) |
| 02 | Driver Station digital inputs | 8-bit bit field (each bit corresponds to a digital input) |
| 03 | Driver Station digital outputs | 8-bit bit field (each bit corresponds to a digital output) |
| 04 | Integer part of battery voltage | 16-bit value Value in hex (i.e. 10 volts is simply 0x10) |
| 05 | Fraction part of battery voltage | 16-bit value Value in hex (i.e. 0.23 volts is simply 0x23) |
| 06 | Robot/Comm. State/Alliance status | 8-bit bit field. Bit values are (as shown in FRCComm.h): Bit 1: reset Bit 2: notEStop Bit 3: enabled Bit 4: autonomous Bit 5: fmsAttached Bit 6: resync Bit 7: cRIOChkSum Bit 8: fpgaChkSum |
| 07 | Error byte | 8-bit bit field. Bit values are unknown. |
| 08 | Upper 2 digits of team number | 8-bit integer |
| 09 | Lower 2 digits of team number | 8-bit integer |
| 0A-11 | Driver Station version | ASCII-encoded string |
| 12-17 | Padding | Always appears to be zero |
| 18-19 | Reply packets sent | 16-bit value Byte order seems flipped from similar counter at 00-01. Byte 18 fills up and rolls over into Byte 19 |
| User Data Follows: |  |  |
| 1A | Fresh Packet number | 8-bit integer (assumed to be unsigned) Increases every time packet contains new user data TODO: Test rollover behavior |
| 1B-1E | User string length | 32-bit integer (assumed to be unsigned) Length of user string to follow. |
| 1F – (Directly after user string length) | User string | ASCII-encoded string |
| (Directly after user string) | Error string length | 32-bit integer (assumed to be unsigned) Error string length |
| (Directly after error string length) | Error String | ASCII-encoded string |
| (Directly after error string) | User data length | 32-bit value (assumed to be unsigned) Length of user data |
| 6D-72 (Directly after user data length)-3F1 | User Data | Data output from robot program |
| Error Checking: |  |  |
| 3F2-3F9 | CRC-32 | crc32 checksum with standard IEEE polynomial 0x04C11DB7 following 4 bytes of padding |