Network Settings for FRC Control System

Non-Classmate development computer:

IP Address:		10.xx.yy.6

Subnet Mask:		255.0.0.0

Default Gateway: 	Blank

Classmate:

Classmate IP settings are set by updates. They are (apparently, based on above settings and current, after-update settings on classmate):

IP Address:		10.xx.yy.5	(known for sure)

Subnet Mask:		255.0.0.0

Default Gateway: 	Blank

Instructions for setting up wireless connection to router for classmate:

Connect to the router wired. No instruction is given to change any wireless IP settings, so they should seem to be set to “obtain an IP address automatically,” the default. Then click on the wireless icon in the notification area (next to clock), select the router, and click connect. Then disconnect the wired connection by unhooking the cable. You should now be set to use wireless.

This has been simply tested and the Classmate does not connect to the robot.

Router: Linksys WRT610N Router

Veteran router is different, rookies have WRT160N. (Veteran router is not sold anymore, apparently there are some spec differences that don’t matter)

IP Address:		10.xx.yy.4

Subnet Mask:		255.255.255.0		(selected from a drop down menu)

Bridge: Linksys WGA600N Gaming Adapter/Bridge

IP Address:		10.xx.yy.1

Subnet Mask:		255.0.0.0

Default Gateway:	10.xx.yy.4

Username:		admin

Password:		admin

CompactRIO:

The device name setting in the Imaging tool can be set to anything. Just set it to Team xxyy to be consistent. You should really not image the cRIO over wireless because a loss of connection is possible, risky, and very disastrous.

IP Address:		10.xx.yy.2

Camera: Axis 206 Network Camera

IP Address:		192.168.0.90

Username:		root

Password:		admin

PC IP Address:		192.168.0.XX		(used for accessing camera manually, XX is

something not in use, 1-255)