What you really need to know if you are the Team Programmer (Java)

If you are the team programmer, you know or will soon know how little time you have to complete a given task, and download your new code. But you will need to be able to revert to your old code on a moment’s notice.  This programmer training guide will help.

You should try to get build team to tell you what they are doing as soon as possible, so you can start working on your code! Some things seem simple until you try to get the robot to do them. (*Section on what they might tell you, and how you can do it AKA Translating from build speak to Java*)

A Robot or computer always does exactly what you want it to do, but the tricky part is figuring out where you told it to do that.

# 10 Things to keep in mind

- Don’t reinvent the wheel.

- Save often, and keep versions of your code

- Before you code, think about how the code must be set up, but don’t get into the nitty-gritty stuff. It’s great to diagram out what you want your code to do (flowchart style) Like the outline of a paper, but for your code

- You will have to debug your code. It won’t work the first time. Make it easy for yourself and make your code readable, use names that make sense like LEFT_DRIVE for the left drive motor instead of motor1.

- If you have to think about it to write the code, leave a comment that explains the thought process and the way that code will work.

- Motors don’t stop when they don’t get any more commands, they continue to run at the last speed they were given – if you want a motor to stop, set it to 0, and if you are in a loop that doesn’t update your drive motors, you will continue at the last heading.

- Before you change your code, make sure that all of your mechanical connections are right. If you expect the Arm to be plugged into port 3, your code won’t work if it’s in the wrong port

- Asking for help means that you are intelligent enough to realize that you don’t know everything yet. “Good judgment comes from experience. Experience comes from bad judgment”- Jim Horning. Sometimes, you don’t have the time to spend, so ask for someone with good judgment. (Mainly at competitions)

- Tabbing your code (indenting with the French braces) is very helpful for debugging and reading.

- KEEP IT SIMPLE! If you can think of 2 ways to do something, do the simpler of the two. Just like with building, if simple works, use it!!!

# What you NEED to know:

- Default code use( page4)

- Set IP and Reimage(page3 ) and Downloading your code (page5 )

- Creating your own Drive code(page 12)

- Basic Custom Control (Joystick to Motor) (page 11)

- Set up your radio/Wireless Network (yes this isn’t exactly “programming” but you will probably be responsible for it) (page 5)

# What would be fantastic to know:

- Basic Steps to Problem Solving(page 15)

- How to version your code – (Keep version of your working code)

- What  certain Errors mean (page 13)

# What would be really good to know:

- Control a motor with a sensor

- Compressor and pneumatics

- A button to a relay

- Setting a motor to run for a specified amount of time or until something happens “Do until” and “Wait”

- How to program an autonomous mode

# What might be helpful to know:

- Turning on a light when a  button or other event happens

- What exactly an object is

# What this Guide shows

- References to other helpful websites and resources

  - The biggest robotics forum for FIRST: chiefdelphi.com

  - FIRST’s Website: usfirst.org

    - 2010 Benchtop Test

    - 2010 Wiring Diagram

  - 341’s Team in a Box

  - Link: java documentation

    - WPI’s site for Java first.wpi.edu/FRC/frcjava.html

    - The 2010 Getting started with Java Manual

      - Includes how to Install Netbeans and the FRC Imaging tool (pages 3 to 5)

      - Differences between C++ and Java (p 21) – This is helpful if you find code that accomplishes your task, but it is written in C++

- Default code use (page 4)

  - Opening Default Code (page 4)

  - Changing which motor ports (page 4)

  - Inverting a motor (page 4)

  - Adding in an extra joystick-> motor Or servo control (page 4)

- How to use a timer to wait to execute code (So you can keep driving) (page 16)

- Set IP and Reimage(page 3)  and Downloading your code(page 5) – explained here and external link

- Set up your radio/Wireless Network (page5)

- Basic Vocab you should know (page 7)

  - Robot vocab (page 7)

  - Programming vocab (page8 )

    - Syntax (page 9)

    - Comparisons(page 8)

    - Structures and Functions–loops and conditionals (page 9)

Creating your own Drive code (page 12)

- Adding in an extra joystick-> motor Or servo control (page 4)

- Basic Steps to Problem Solving (page 15)

- Errors

  - Complier errors – these are Syntax Errors

  - Robot doesn’t work

  - Structures and Functions–loops and conditionals (page 9)

- Setting up your own motors, sensors, and constants (page 13 )

  - Motors (page 13)

    - Jaguars

    - Victors

  - Relay (page 14)

  - Sensors (page14 )

    - Limit switches, UltraSonics, Gyro, Potentiometers

  - Joysticks (page 15)

  - Variables (page 15)

  - Constants(page 15)

# Set IP and Reimage

If you’ve done the Benchtop test or used the driver station before, Skip to step. Note that these steps are paraphrased from the 2010 Section 2 of the Control Manual. It is HIGHLY RECOMMENDED that you use FIRST’s documentation. But just in case, here is our version.

- If you haven’t used the Classmate yet, the process for setting it up is in the 2010 Control Manual, Section 2 page 29. This only needs to be done once.

## Set Static IP address of computer

This information can also be found on page 31 of the 2010 Control Manual, Section 2.

- Start » Control Panel » Network Connections » Local Area Connections

- Under the General Tab, Select Internet Protocol (TCP/IP) and click properties

- Select the “Use following IP address” Option

- If you are using the Classmate, set the IP address to 10.xx.yy.5 otherwise set it to 10.xx.yy.6  (NOTE: xxyy is your team number. If your team number is less than 4 digits, add zeros in front until it becomes 4 digits. Ex. Team 1’s classmate has an IP of 10.00.01.5 *which will simplify to 10.0.1.5*)

- The subnet mask text box defaults to 255.0.0.0. Use this value

- Click Okay on both the Internet Protocol (TCP/IP) and Network Connections dialog boxes

## Reimaging the cRIO (not necessary or desired each download)

This information can also be found on page 33 of the 2010 Control Manual, Section 2.

- Use an Ethernet Crossover cable connected to port 1 of the cRIO and to your computer.

- Open FRC cRIO Imaging Tool (if it’s not a shortcut, you should make one)

- Select your cRIO from the “Select cRIO Device” table (The cRIO must be on, and connected to the computer. If you get an error in a pop-up that says “No CompactRIO Devices were found. Verify the network connection.” Check that the cRIO is on and connected, then click “Rescan now”

- In the Development Environment section, select Java. (This guide explains Java coding. If you are coding in a different language, this guide isn’t for you)

- Place a checkmark in the Format Controller box

  - Within this section you can restore an image,  update the cRIO with a new name, or change the team ID.

- From the “Select Image” list,  select the most recent image (the 2010 images were named FRC_2010_xx.zip  -- xx stated which version it is)

- Enter what you want your cRIO to be called in the Device Name box.

- Enter your team number in the Team ID field – it automatically sets your cRIO address to be 10.xx.yy.2

- Click Apply. It’ll take a couple minutes, so don’t play with the network cable or turn off the power.

- Once the Reconfiguring device states that the cRIO Image has been successfully updated, turn off the cRIO and restart it. Downloading your code is the next step.

# Default code use

## Opening Default Code

- Open up Netbeans and open a New Project  (or press Ctrl+Shift+N)

- A file tree should come up. Expand the Samples folder then click on FRC Java (the one within the Samples file)

- On the right there is a list of the Projects contained within the folder,  select “DefaultCodeProject.zip”  and hit next.

- ****IMPORTANT**** Change the name of the project – this will help you Identify which code you are working on. You could change it to “DefaultCodeTest” or something of the sort.

- Look at the project location and change it if you want to save your code elsewhere.

  - I highly recommend that you create a folder at the called robotics at the base level of your hard drive (C drive). Have an additional folder within the Robotics folder for code (maybe an additional level for Versions) Your file path will look similar to C:\Robotics\Code

- The new project is automatically set to the main project, and you should download your code – Instructions  → Downloading your code

## Changing which motor ports

- Open default code project.

- If you want to change the motor ports for the drive train, go to line 114.

- The set up order is (Left,Right) for 2 motors, and (Front Left, back Left, Frontright, Back Right) for 4 motors

- So if you have your motors plugged into ports 5 and 6, line 114 should say “m_robotDrive = new RobotDrive(5,6);” (assuming 5 is the left motor and 6 is the right motor)

## Inverting a motor

- If you need to invert your motor, find out which port it is plugged into.

- Press enter to make a line just below where you set up your motor. (line 114 in default code)

- Add the line “setInvertedMotor(/* YOUR MOTOR PORT’S NUMBER HERE (like 5) */, true);

## Adding in an extra joystick-> motor Or servo control

- You need to tell the computer what you will be calling the motor (set up the object)

- Set up the Joystick

- Then add the line “m_arm.set(m_operatorStick.getY());” in TeleopPeriodic (line 225 -294)

  - If you aren’t using the Default code, add it just below your drive code

- **If you want Servo control the line would be “m_arm.set((m_operatorStick.getY()+1)*.5);”

  - Servos have a range of 0 to 1, but joysticks and motors have a range of -1 to 1.

    - The general formula for converting a number from one range to another is “newValue = newMax +(newMax-newMin)*(oldValue-oldMax)/(oldMax-oldMin)” (Taken from the Chief Delphi post by FRC4ME in the thread “Servo Control with a Joystick”)

# Downloading your code

- Open up your Netbeans project and turn on your cRIO.

- Connect your computer to the cRIO (port 1) via a crossover Ethernet cable. (You can download wirelessly, but there can be complications. For now, tether the robot)

- Save your code. Now would be a great time to make another version of your code.

- Right click on your project name and click “Set as main project.” This will make the project name bold.

- Check to make sure that you don’t have any red stop signs with explanation points (PICT) in your code.

  - If you do go to Code Errors for more information.

    - You should turn off the cRIO to save power, and debug.

- Then click the green run button (or press F6).

  - If you get a “BUILD FAILED” message, there is something wrong.

    - If the last lines that are in black on the build output are “Connecting FTP @10.xx.yy.2” (remember that xx.yy will be replaced with your team number) and it says “Java.net.SocketException: Connection Reset” then you are not connected to the cRIO. Make sure the cRIO is on, and the Ethernet cord is in port 1 of the cRIO and in your Ethernet port on the computer.

- Open up the Driver Station (press windows+ L and click on the Driverstation icon on the classmate) and test your code.

# Set up your radio/Wireless Network

As with many of the other sections, this information is also available on the FIRST site. The link is http://usfirst.org/uploadedFiles/Community/FRC/Game_and_Season__Info/2010_Assets/2-2010FRCControl%20System-Getting%20Started-Rev-0.7.pdf

FIRST 2010 FRC Control System Manual: Section 1 – Introduction, Rev1 Page 45 of 58

Set your computer to receive it’s IP via DHCP.

Start » Control Panel » Network Connections » Local Area Connections

Under the General Tab, Select Internet Protocol (TCP/IP) and click properties

Set it to Obtain IP address Automatically

Connect the computer’s Ethernet port on the Driver Station to any of the 4 Ethernet ports (NOT THE ONE LABELED INTERNET) on the router using a Ethernet cable.

Connect the Bridge to any of the remaining 3 Ethernet ports on the router using another Ethernet cable.

Note: If during this process, your computer prompts you to set up security, your computer probably has “Wi-fi protected setup” which will automatically set up the security settings on the router. If this happens, you will need to either change the settings on the bridge to match the router settings, or you will need to disable the security on the router. More information about Wi-fi protected setup can be found at http://wi-fi.org/files/WFA%20Wi-Fi%20Protected%20Setup%20FAQ.pdf .

Open a web browser and navigate to the IP of the bridge(type in the IP address as the web address), the default settings are: a. IP: 192.168.1.250

Username: admin

Password: admin

Select "Basic" from the menu across the top of the webpage.

Select "Wireless" to open the configuration page for the wireless settings. The configuration page is shown below:

Change the "Wireless Network Name" to your FRC Team Number, no quotation marks.

Configure all other settings to match those shown above. a. NOTE: When using the WRT160N the “802.11 Band” option should be set to 2.4GHz

Click “Save Settings”

When asked to reboot now or later, choose “Reboot Now”

Login again. Username: admin   Password: admin

Select "Basic" from the menu across the top of the webpage.

Select "Network Settings" to open the configuration page for the wireless settings. The configuration page is shown below:

d. Configure the LAN Settings to match those shown above. In the IP Address and Default Gateway lines, replace “XX.YY” with your FRC Team Number.

Click the “Save Settings”

When asked to reboot now or later, choose “Reboot Now”. The login page will not reload at this point due to the IP address mismatch of the PC and bridge.

# Basic Vocab

## Robot vocab

#### Arcade driving

1 joystick controls the drivetrain. Pushing it forward makes the robot go forward, pushing it to the left makes the robot go left. (Generally people are better at first at arcade, but with only a little bit of practice, they are better at tank)

#### CIM

the most common motor for the drive train. You can only use a limited (FIND #) of these per robot

#### cRIO

the brain/main computer of the robot. Has many modules.

#### Encoder

a digital sensor that can be used to measure rotations (and distances) (plugs into the digital I/O on the digital sidecar)

#### Fisher Price

Small highspeed motor. Usually comes with big plastic gearbox

#### Gyroscope (aka Gyro)

analog sensor that can measure turning (of the robot) (can be used on omni-bots to make pushing the stick forward always make the robot go away from you)

#### Jaguar

a variable speed motor controller. (larger size than the Victors, and stylized) Plugs into the PWM OUT on the Digital sidecar

#### Jumper

tiny piece of plastic covered metal – connects 2 pins. Used on the Analog Breakout board for getting battery voltage, and for servos.

#### Limit Switch

a digital sensor used to see if something is touching a certain point.

#### Potentiometer (aka pot)

an analog sensor (plugged into the analog breakout board) that will give different values for different angles – but has a set range of motion.

#### PWM

3 colored wire (usually white-red-black) this will be your most of the wires coming out of the cRIO

#### Spike

an on/off power switch for a motor (a relay). The PWM plugs into the relay ports on the digital sidecar.

#### Servo

a small motor with a built in potentiometer. Has a set range of motion. Plugs into the PWM OUT on the digital sidecar but a jumper must be added on the 2 pins next to it.

#### Tank driving

Uses the y axis of 2 joysticks to drive the robot. Each joystick control half of the drivetrain. Pushing the left stick forward will turn on only the left motors, and send the robot to the right. Pushing both sticks forward will make the robot go forward.

#### Victor

a variable speed motor controller. (smaller size than the jaguars, and more block-like. Doesn’t come in the new kit) Plugs into the PWM OUT on the Digital sidecar

#### Window Motor

slow powerful motor. Usually runs off a relay.

## Programming vocab

#### Camel Case

the convention(tradition) of writing the first word in a series lower case, then the first letter of each following word uppercase. itShouldLookLikeThis

### Comparisons

There are two ways equal signs are used. (a=b) will set a to the value of b, while a == b will check to see if the a and b are equal. (a==b) will return a Boolean value of true or false.  Possible error message “Incompatible types” – you don’t have the same type of variable being compared. Possible warning:  “Possible loss of precision” This means 2 things. 1. you are setting a more precise (double) to a less precise one (int). 2. You are setting a to have the value of b, not testing if a and b are equal

! – Explanation points are the programming “not”  (a!=b) will return true if  a doesn’t have the same value as b. It inverts the value of the Boolean. (A value that would be false returns true, and vise versa)

a>=b –If a is greater than or equal to b, it will return true

a<=b – If a is greater than or equal to b, , it will return true

a>b  --If a is greater than b, it will return true

a<b – If a is less than b, it will return true

Keep in mind that (a>=b) will be the functionally the same as !(a<b)

#### Constants

In Java, variables and constants are created almost the same way, except a keyword “final” is put in front of the variable type. Traditionally, constants are named in ALL_CAPS and they cannot have spaces in the name.

final double PI = 3.14159265358979323;

#### Comments

Comments don’t run on the robot, they only help the programmer. You can comment out non-functional code or write a comment to describe a line. There are 2 types of comments: single line comments and multiline comments. A single line comment starts with a double forward slash, and comments out the rest of the line. A multiline comment starts with a forward slash then an asterisk (/* ) and ends with an asterisk and a forward slash (*/).

Not a comment   // this is a comment                         so is this

This isn’t

/*this is a comment                                                                           Still a comment

Still a comment */                   but this isn’t

#### French Braces {}

They package code. They enclose the code executed during a loop or an if statement. They always come in pairs, make sure you have one on each side of your block of code. Possible error: Illegal start of Expression or Parsing error. Both mean you have the wrong number or placement of Braces. It’s often helpful to print out your code and draw lines connecting corresponding braces to see where you went wrong

#### Function/Method

A set of lines of code that can have parameters and return a value. It’s a good idea to make a function when you want to have a bit a code you call (or do) repeatedly.

There are many built-in functions (such as tankDrive(left

#### Objects

you don’t need to know very much about Objects, but you have to be able to create and use an object. You will be setting up your motors, relays, servos and everything through objects.

When you want to change or get information (through a function) you put the object name, then a period then the function (with all parameters)

#### Parameters

The inputs in a function. Think of it as the information it needs to know before it can do anything. (Like doing a homework assignment, you need to know the page it’s on before you can actually do it). Parameters are separated by a comma.

#### Semicolons

Semicolons are the periods of Java. They finish a statement.

#### Syntax

Java is a language, and syntax describes the grammar. If you get a syntax error, there is something wrong with the way you set up your line of code. In more depth in the Syntax section (here)

#### Variables

The value of this can change throughout the program

Boolean

only true or false

Double

any number, includes decimals

Int

almost all integers(no decimals allowed). Odds are high that the only time you’d need a greater range of numbers is if you were using a high resolution encoder. You would use a “long” in this case.

String

a “string” of numbers, letters and symbols. You are unlikely to use this variable type except in System.out.println(toPrint)

### Structures and Functions

Conditional – this is the statement that will be evaluated – it will have a Boolean (true/false) value, and can use Comparisons  such as =, >=, >, <=, < or !=.

If Statements – The most important conditional.  You can have if’s without an else or an else if, but not the other way around. You’ll get a syntax error if you try.

| if (/* INPUT WHAT YOU WANT TO BE TRUE */) { // THIS IS WHAT WILL HAPPEN if it’s true } else if(/* Second thing to test*/) /* Will only be tested if the original condition is false*/ { /* Will happen if the first condition is false, and the second is true*/ } else /* Notice that this doesn’t have a condition */ { // will happen if both conditions are false } | if(a>b) { System.out.println(a); //prints the value of a } else if(a<b) { System.out.println(b); //prints the value of b } else { System.out.println(“a and b are equal to “ a); } /*This will print out the value of the greater of the 2 variables, or state that they are equal and give the value*/ |
| --- | --- |

While loops. – the most important loop. It will execute the code within the French braces while the condition is true

| while(/*conditional is true*/) { // this code will be done while the conditional is true. } | while(isEnabled()) /* this code will run only while the robot is enabled ---- IT WILL ALSO ONLY RUN IF THE ROBOT IS ENABLED WHEN THIS CODE IS RUN (not if you enable later) */ { m_robotDrive.tankDrive(m_leftStick, m_rightStick); } // the above line is straight from the Default code |
| --- | --- |

# Calling a Function

Functions can have parameters, and you need to include the right data type for each. – there is a list of the functions and it is found at http://livemyst.comli.com/wpilibj/doc/index.html and the ones you are most likely to use are found below (with a brief explanation)

| Type | Returns | Function name | Parameters |  | Notes |
| --- | --- | --- | --- | --- | --- |
| private public | int double string boolean void | ExampleFunction( | int First, double Second); ); |  | If it returns void, it doesn’t give any value back. Don’t try to set it equal to something. |
| public void set(double speed); |  |  |  | You use this one by typing in the name of your motor (example here is m_shoulder) Range of values is -1 to 1 m_shoulder.set(0); // Stops the shoulder motor Somewhere above in your code, you must have initialized m_shoulder. See Setting up your own motors, sensors, and constants |  |
| public void set(double value); public void setAngle(double degrees); |  |  |  | Used for setting the position of a servo. (The set function acceots values from 0 to 1) If the servo has a max angle of X and a min angle of Y, values greater than X will result in X being set and angles less than Y will result in Y being set |  |
| public double get(); |  |  |  | You use this function to get the speed of the motor specified. Notice there are no parameters. speed = m_shoulder.get(); |  |
| public boolean get(); |  |  |  | Gets a digital input from cRIO (Must have set up m_frontLimit and hit) hit=m_frontLimit.get(); |  |
| public boolean getDigitalIn(int channel); |  |  |  | Gets a digital input from THE DRIVERS STATION final int SHOOT_BUTTON = 2; boolean shoot = false; shoot=m_ds.getDigitalIn(SHOOT_BUTTON); |  |
| public int getValue(); |  |  |  | Get an analog input from the Robot. Unless you are using a custom sensor, there are function calls for most sensors. turning =m_analogCustom.getValue(); |  |
| public double getAngle(); |  |  |  | Gyro – returns the heading heading = m_gyro.getAngle(); |  |
| public double getRangeInches(); public double getRangeMM(): |  |  |  | You set up at the which units you use at the beginning. distance = m_frontUltrasonic.getRangeInches(); |  |
| public void set(int value); |  |  |  | Setting the direction on a relay. The values for direction you can use are Value.kOn Value.kOff Value. kForward and Value.kReverse -- you can only use Value.kOn if the relay is set to go in only one direction //make sure you import edu.wpi.first.wpilibj.Relay.*; |  |
| public void tankDrive(double leftSpeed, double rightSpeed); |  |  |  | Allows you to drive tank style. if you named your robotDrive object m_robotDrive and you create and set leftSpeed and rightSpeed to your axises, you will call it by writing m_robotDrive.tankDrive(leftSpeed,rightSpeed); |  |
| public double getAxis(int axis); |  |  |  | This is used to get a joystick axis. you can use AxisType.kY, AxisType.kX, AxisType.kTwist, AxisType.kThrottle, and AxisType.kZ if you import edu.wpi.first.wpilibj.Joystick.AxisType; |  |
| public void arcadeDrive(double speed, double curve, boolean spin); public void arcadeDrive(GenericHID stick); |  |  |  | Used with RobotDrive 1.Speed is usually the y-axis of the stick and curve is the x. If you want the robot to be able to spin in place(zero point turn) spin should be true 2. Just set up the joystick, and input it (the object) here |  |

By writing the name of the thing(motor, joystick, sensor etc.) then putting a period, and then the function name, you are telling it which motor to act on.

# Basic Custom Control (Joystick to Motor)

You create a new project (Simple Robot)

There are 2 functions within the simple robot project: autonomous and operatorControl.

On line 11, there is the import line.

You will want to include (at least) these files See

import edu.wpi.first.wpilibj.SimpleRobot;	/* this one is included when you open the project. Don’t delete it */

import edu.wpi.first.wpilibj.RobotDrive;		/*allows you to use pre-made functions for driving*/

import edu.wpi.first.wpilibj.Joystick;		/*allows you to get and use data from joystick */

import edu.wpi.first.wpilibj.Joystick.*;		/* allows you to use kY and other axis */

import edu.wpi.first.wpilibj. Jaguar; 		/*  allows you to control your motors---If you are using Victors instead of Jaguars, this line should say  “import edu.wpi.first.wpilibj.Victor;”   **** IF YOU ARE USING BOTH VICTORS AND JAGUARS, YOU NEED BOTH LINES*/

You need to tell the robot what you are calling everything and what type of thing it is.

To do that, go to the line after “public class FromScratch extends SimpleRobot { “ **NOTE: I named my class FromScratch. Your name will probably be different. Whatever you put in the “Robot Class” when you named the code will be in place of FromScratch** and press enter a couple times. This is where you should set up your constants and robot parts.

RobotDrive m_robotDrive     =new RobotDrive(1,2); /*Set for 2 motor drive, Left = port 1, right = port 2*/

Joystick m_leftStick	 = new Joystick(1);       /*joystick called m_leftStick plugged into Port 1*/

Joystick m_rightStick 	= new Joystick(2);	   /* joystick called m_rightStick plugged into Port 2 */

Joystick m_operatorStick	 = new Joystick(3);   	   /*joystick called m_operatorStick plugged to port 3*/

Jaguar   m_arm	= new Jaguar(3);	   /*joystick called m_arm plugged into port 3*/

double arm_speed 	=0;

If you haven’t edited it at all yet(I’d have to wonder why you are reading this if you haven’t), operator control starts on line 31, but once you have put in the imports, and you have set up your motors, it will be on a different(later) line. Go to the line after “public void operatorControl(){ “ and make a while loop.

while(isOperator Control() && isEnabled()) 	/*here’s a great time to get started on TABBING*/

{

m_robotDrive.tankDrive(m_leftStick, m_rightstick);		// Drive tank style

// Now I am going to hook up a motor to a joystick. (In this case, the y axis.)

arm_speed = m_operatorStick.getAxis(AxisType.kY);  /*if you want the x axis, replace kY with kX*/

m_arm.set(arm_speed);

} // end while Operator control

# Creating your own Drive code

There are several main types of driving that most teams use. For basic drivetrains, there is tank or arcade. There are also omni drive systems, the most common being Mecanum (a type of wheel). All of these types have premade code for driving if you use robot drive.

Sometimes your motor would run in the opposite direction that it should, so you need to invert it.

Constructors:

RobotDrive(int leftMotorChannel, int rightMotorChannel)

RobotDrive(int frontLeftMotor, int rearLeftMotor, int frontRightMotor, int rearRightMotor)  /*It doesn’t really matter what motor is in front and which one is in the back */

These functions are set up the same way as shown in Calling a Function

public void arcadeDrive(double moveValue, double rotateValue, boolean spin);

public void arcadeDrive(GenericHID stick);	/*if you set it up as a joystick, it will still work here */

public void drive(double speed,  double curve);

public void holomonicDrive(double magnitude, double direction, double rotation); /*Holonomic drive is Omni drive for the Mecanums */

public void setInvertedMotor(int motor, boolean isInverted);

public void tankDrive(double leftSpeed, double rightSpeed);

public void tankDrive(GenericHID leftStick, GenericHID rightstick);

If you want to have a drive set up different than any of these (for example, crab drive) you will want figure out how each of the motors will be getting their information. Crab drive has one (or more) motors which turn the individual wheels. You would probably want an encoder or potentiometer on these, so you can easily “zero”(reset to straight) the wheels.

# Errors

## Syntax

Semicolons: Java :: periods:English

If you have a generic syntax error, check the line before it for a semicolon or other ending statement (like the correct amount of closing parenthesis or brackets)

# Setting up your own motors, sensors, and constants

Objects are set up in the same way every time. Up near the top, you need to include the corresponding file. (import edu.wpi.first.wpilibj.ObjectType;)

Then you decide what you are going to call it, write a line like ObjectType name;  This will be included after the line that starts “public class” and ends with a {  but before the constructor/initialize function

Then within the constructor/initialize function  you will add the line “name = new ObjectType(/*parameters*/);

In the default code, they set up a convention that your robot parts are prefaced with m_ and then written in Camel Case. So the left motor is usually called m_leftMotor

## Motors

Find out which speed controller you are using (Victor or Jaguar) and go to the correct section

### Jaguars

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Jaguar;”

In this example, the motor is named m_jag and it is in port 7 – “Jaguar m_jag;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_jag = new Jaguar(7);”

### Victors

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Victor;”

In this example, the motor is named m_vicky and it is in port 8 – “Victor m_vicky;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_vicky = new Victor(8);”

### Servos

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Servo;”

In this example, the motor is named m_twitchy and it is in port 6 – “Servo m_twitchy;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_twitchy = new Servo(6);”

## Relay

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Relay;” and “import edu.wpi.first.wpilibj.Relay.*;”

In this example, the relay  is named m_blinky and it is in port 2 – “Relay m_blinky;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_blinky = new Relay(2, /*DIRECTION*/);”

Direction will be one of 3 things : Direction.kForward OR Direction.kReverse OR Direction.kBoth

If you only need to go one direction, set it as either Direction.kForward OR Direction.kReverse and then use Value.kOn OR Value.kOff . It makes it simpler

If you need to go both directions, you will have to use Value.kForward and Value.kReverse

## Sensors

### Limit switches

These are Digital Inputs.

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.DigitalInput;”

In this example, the limit switch is named m_magLimit and it is in port 1 – “DigitalInput m_magLimit;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_magLimit = new DigitalInput(1);”

### UltraSonics

This is one digital input, and one digital output.

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Ultrasonic;”

In this example, the ultrasonic  is named m_screech and the output is in port 3 and the input is on port 7 – “Ultrasonic m_screech;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_screech = new Ultrasonic(3,7);”

### Gyro

This is an anolog sensor.

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Gyro;”

In this example, the relay  is named m_spinny and it is in port 2 – “Gyro m_spinny;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_gyro = new Gyro(2);”

### Potentiometers

This is an analog sensor

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.AnalogChannel;”

In this example, the potentiometer  is named m_pot and it is in port 1 – “AnalogChannel m_pot;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_blinky = new AnalogChannel(1);”

## Joysticks

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Joystick;”  and “import edu.wpi.first.wpilibj.Joystick.AxisType;”

In this example, the joystick  is named m_operatorStick and it is in port 3 – “Joystick m_operatorStick;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_operatorStick = new Joystick(3);”

## Variables

Choose which type of Variables you want (Boolean , Int, String, or Boolean)  (also usually in Camel Case)

Then it is set up

variableType name;  //--- UNINITIALIZED. BAD Choice.

variableType name = startingValue;

ex.

double price = 1.23;

int count = 0;

string toPrint = “Hello world!”;

boolean fun = true;

## Constants

Set up as a variable with the keyword final (usually in all caps)

So it would be

final double PI =3.14159;

# Basic Steps to Problem Solving

- Recognize it can be done

  - “Impossible only means that you haven’t found the solution yet.” (from thinkexist.com)

- Think about the way you would do it if you had only the inputs you can give a robot

  - Ex. If you need to figure out how to follow a wall, close your eyes, and think about how you would do it blindfolded. (trail your hand along the wall

  - You CAN make there be more sensors on the robot. It’s okay if you can’t do it with what you are given.

  - Think about how the robot will have respond to its inputs and its limitations

  - If you want it to go in a circle when a button is pressed, make sure it can physically go in a circle.

    - Make sure that you only set your motors one time per loop. If you set it several times, you will be confused and your bot won’t work right. – which is why it is so important to use good variable names and constants.

Psuedo-code is my favorite coding tool. You just write out the main points of your program - don’t worry about syntax or how to read a sensor. Or even how you will implement something big. Get down order and choices. Then move on to the next piece that you skipped over. NOTE: this isn’t actually code. It’s usually scribbled on random pieces of paper. It just helps you figure out what to do.

//needs to drive in a circle when a button is pressed

if(button)

{

circle();

}

else

{

driveNormally();

}

- If you can’t figure it out (whether a small piece of code or the actual problem) ask for help. You can post on Delphi or on FIRST Forum.

- I make models, walk through hallways and wave my arms around sometimes when I am trying to figure things out. Don’t feel stupid if you want to put yourself in your robot’s “shoes.”  It’s a helpful technique.

- Although timing is the technique most people fall back on, (for autonomous coding)  it works and it works decently. If you want a more reliable technique (where the battery voltage won’t mess up your timing) try something sensor based.

# How to use a timer to wait to execute code (So you can keep driving)

This is a more advanced topic. It assumes that you understand drive code.

You will have to add “ import edu.wpi.first.wpilibj.Timer;”  and initialize and start a timer. (it’s a good idea to call it something relevant to why you need one ex. timeGrab)

Instead of using a wait function to wait for your delay, you can use a timer and if statements.

The code below also shows how we tackled the problem of doing something ONLY when the limit switch was first pressed.

/* This was actual code from 2234’s robot in 2010. We needed to add in a delay because we would push forward with pneumatics, but the string we were trying to latch onto with a compound bow trigger would move slightly. Waiting for250 ms made the grip much better*/

/*

the servo is called m_trigger.

SHOOT is the input (and a constant) on the Cypress FirsTouch board that our button was connected to

RELEASE and GRAB are constants that we found through experimentation.

delay_grab is the name of the timer

m_canReach is the name of the limit switch (which tests to see if we are in reach to grab the string)

*/

if(m_canReach.get())

{

if(!m_ds.getDigitalInput(SHOOT)) /*this particular switch defaults to true (this tests if the switch is pressed)—“if(!m_ds.getDigitalInput(SHOOT))” is same as “if(m_ds.getDigitalInput(SHOOT) == false)” */

{

m_trigger.set(RELEASE);

}

else

{//shoot button not pressed

if(pressed==false)

{// this means the limit switch was just pressed

delay_grab.start(); // start counting

pressed = true;

}

else

{ // the limit switch has been pressed for a little while

if(delay_grab.get() >= TIME_DELAYED)

{

m_trigger.set(GRAB);

delay_grab.stop();

delay_grab.reset();

}

} // end else the limit switch has been pressed for a while

} // end shoot button not pressed

}// end limit m_canReach is pressed

else // not touching, therefore the trigger should be open. And reset the timer.

{

m_trigger.set(RELEASE);

pressed = false;

delay_grab.stop();

delay_grab.reset();

}//end else m_canReach

You can use a similar thought process (do something if it is greater than a predetermined time, and something else if it is less) to do anything you need.  Just make sure that you stop and reset when you are done, and you start the Timer when you need to.