What you really need to know if you are the Team Programmer (Java)

If you are the team programmer, you know or will soon know how little time you have to complete a given task, and download your new code. But you will need to be able to revert to your old code on a moment’s notice.  This programmer training guide will help.

You should try to get build team to tell you what they are doing as soon as possible, so you can start working on your code! Some things seem simple until you try to get the robot to do them. (*Section on what they might tell you, and how you can do it AKA Translating from build speak to Java*)

A Robot or computer always does exactly what you want it to do, but the tricky part is figuring out where you told it to do that.

# 10 Things to keep in mind

- Don’t reinvent the wheel.

- Save often, and keep versions of your code.

- Before you code, think about how the code must be set up, but don’t get into the nitty-gritty stuff. It’s great to diagram out what you want your code to do (flowchart style). Like the outline of a paper, but for your code.

- You will have to debug your c ode. It won’t work the first time. Make it easy for yourself and make your code readable, use names that make sense like LEFT_DRIVE for the left drive motor instead of motor1.

- If you have to think about it to write the code, leave a comment that explains the thought process and the way that code will work.

- Motors don’t stop when they don’t get any more commands, they continue to run at the last speed they were given – if you want a motor to stop, set it to 0, and if you are in a loop that doesn’t update your drive motors, you will continue at the last heading.

- Before you change your code, make sure that all of your mechanical connections are right. If you expect the Arm to be plugged into port 3, your code won’t work if it’s in the wrong port

- Asking for help means that you are intelligent enough to realize that you don’t know everything yet. “Good judgment comes from experience. Experience comes from bad judgment”- Jim Horning. Sometimes, you don’t have the time to spend, so ask for someone with good judgment. (Mainly at competitions)

- Tabbing your code (indenting with the French braces) is very helpful for debugging and reading.

- KEEP IT SIMPLE! If you can think of 2 ways to do something, do the simpler of the two. Just like with building, if simple works, use it!!!

# What you NEED to know:

- Default code use and Downloading your code

- Creating your own Drive code

- Basic Custom Control (Joystick to Motor)

- Set up your radio/Wireless Network (yes this isn’t exactly “programming” but you will probably be responsible for it)

# What would be fantastic to know:

- Pneumatics

- Starting a Compressor

- Basic Steps to Problem Solving

- Error: Reference source not found

- What  certain Errors mean

# What would be really good to know:

- Controlling a motor with a Sensor

- Error: Reference source not found

- Setting a motor to run for a specified amount of time or until something happens “Do until” and “Wait” – (Which is the same as How to use a timer to wait to execute code (So you can keep driving)

- How to program an autonomous mode

# What might be helpful to know:

- Turning on a light when a  button or other event happens

- What exactly an object is – http://java.sun.com/docs/books/tutorial/java/concepts/object.html has  pretty good description

# Some tips on using this Guide

- The basic topics should be mastered first. If you can’t get your robot to drive, don’t move onto the more complicated bits of code. The first thing any robotics programmer should be able to do is hook up motors to joysticks.  – That IS explained here.

- I have tried to cross-reference this guide extensively. If see an underlined word, it means that it can link to another section of the manual. To follow that link you press Control and click on the words. When you hover over it, it should tell you that.  Unfortunately, I have not yet figured out how to make it go back to where it was before, so if you use a cross-reference, note where you were.

- For those of you who will be printing out this manual, there is a table of Contents on page 3.

- I didn’t go into certain topics (like the camera) because those aren’t needed for the beginning programmer. If you find yourself needing to know something not shown in the guide, it was either more complicated than I had time to explain or a simple combination of a couple things. If you can’t get it, after diagramming out the code and trying to break it down into smaller pieces, you should ask for help.

# References to other helpful websites and resources

- The biggest robotics forum for FIRST: chiefdelphi.com

- FIRST’s Website: usfirst.org

  - 2010 Benchtop Test

  - 2010 Wiring Diagram

- 341’s Team in a Box

- Java documentation

  - WPI’s site for Java first.wpi.edu/FRC/frcjava.html

  - The 2010 Getting started with Java Manual

    - Includes how to Install Netbeans and the FRC Imaging tool (pages 3 to 5)

    - Differences between C++ and Java (p 21) – This is helpful if you find code that accomplishes your task, but it is written in C++

Table of Contents

10 Things to keep in mind	1

What you NEED to know:	1

What would be fantastic to know:	1

What would be really good to know:	2

What might be helpful to know:	2

Some tips on using this Guide	2

References to other helpful websites and resources	2

Set IP and Reimage	5

Set Static IP address of computer	5

Reimaging the cRIO (not necessary or desired each download)	5

Default code use	5

Opening Default Code	5

Changing which motor ports	6

Inverting a motor	6

Adding in an extra joystick-> motor Or servo control	6

Downloading your code	6

Set up your radio/Wireless Network	7

Basic Vocab	7

Robot vocab	7

Programming vocab	9

Comparisons	9

Comments	9

Structures and Functions	10

Calling a Function	11

Tabbing	12

Basic Custom Control (Joystick to Motor)	13

Creating your own Drive code	14

Errors	14

Syntax	14

Setting up your own motors, sensors, and constants	15

Motors	15

Relay	16

Sensors	16

Pneumatics	17

Driver Station	17

Joysticks	18

Variables	18

Constants	18

Pneumatics	18

Starting a Compressor	18

Using A Solenoid (Using Pneumatics)	18

Basic Steps to Problem Solving	18

How to use a timer to wait to execute code (So you can keep driving)	19

Versioning Your Code	20

Controlling a motor with a Sensor	21

Analog Sensor	21

Digital Sensor	23

# Set IP and Reimage

If you’ve done the Benchtop test or used the driver station before, Skip to step. Note that these steps are paraphrased from the 2010 Section 2 of the Control Manual. It is HIGHLY RECOMMENDED that you use FIRST’s documentation. But just in case, here is our version.

- If you haven’t used the Classmate yet, the process for setting it up is in the 2010 Control Manual, Section 2 page 29. This only needs to be done once.

## Set Static IP address of computer

This information can also be found on page 31 of the 2010 Control Manual, Section 2.

- Start » Control Panel » Network Connections » Local Area Connections

- Under the General Tab, Select Internet Protocol (TCP/IP) and click properties

- Select the “Use following IP address” Option

- If you are using the Classmate, set the IP address to 10.xx.yy.5 otherwise set it to 10.xx.yy.6  (NOTE: xxyy is your team number. If your team number is less than 4 digits, add zeros in front until it becomes 4 digits. Ex. Team 1’s classmate has an IP of 10.00.01.5 *which will simplify to 10.0.1.5*)

- The subnet mask text box defaults to 255.0.0.0. Use this value

- Click Okay on both the Internet Protocol (TCP/IP) and Network Connections dialog boxes

## Reimaging the cRIO (not necessary or desired each download)

This information can also be found on page 33 of the 2010 Control Manual, Section 2.

- Use an Ethernet Crossover cable connected to port 1 of the cRIO and to your computer.

- Open FRC cRIO Imaging Tool (if it’s not a shortcut, you should make one)

- Select your cRIO from the “Select cRIO Device” table (The cRIO must be on, and connected to the computer. If you get an error in a pop-up that says “No CompactRIO Devices were found. Verify the network connection.” Check that the cRIO is on and connected, then click “Rescan now”

- In the Development Environment section, select Java. (This guide explains Java coding. If you are coding in a different language, this guide isn’t for you)

- Place a checkmark in the Format Controller box

  - Within this section you can restore an image,  update the cRIO with a new name, or change the team ID.

- From the “Select Image” list,  select the most recent image (the 2010 images were named FRC_2010_xx.zip  -- xx stated which version it is)

- Enter what you want your cRIO to be called in the Device Name box.

- Enter your team number in the Team ID field – it automatically sets your cRIO address to be 10.xx.yy.2

- Click Apply. It’ll take a couple minutes, so don’t play with the network cable or turn off the power.

- Once the Reconfiguring device states that the cRIO Image has been successfully updated, turn off the cRIO and restart it. Downloading your code is the next step.

# Default code use

## Opening Default Code

- Open up Netbeans and open a New Project  (or press Ctrl+Shift+N)

- A file tree should come up. Expand (click the plus sign to the left of the words) the Samples folder then click on FRC Java (the one within the Samples file)

- On the right there is a list of the Projects contained within the folder,  select “DefaultCodeProject.zip”  and hit next.

- ****IMPORTANT**** Change the name of the project – this will help you identify which code you are working on. You could change it to “DefaultCodeTest01” or something of the sort.

- Look at the project location and change it if you want to save your code elsewhere.

  - I highly recommend that you create a folder at the called robotics at the base level of your hard drive (C drive). Have an additional folder within the Robotics folder for code (maybe an additional level for Versions) Your file path will look similar to C:\Robotics\Code

- The new project is automatically set to the main project, and you should download your code – Instructions  → Downloading your code

## Changing which motor ports

- Open default code project.

- If you want to change the motor ports for the drive train, go to line 114.

- The set up order is (Left,Right) for 2 motors, and (Front Left, back Left, Frontright, Back Right) for 4 motors

- So if you have your motors plugged into ports 5 and 6, line 114 should say “m_robotDrive = new RobotDrive(5,6);” (assuming 5 is the left motor and 6 is the right motor)

## Inverting a motor

- If you need to invert your motor, find out which port it is plugged into.

- Press enter to make a line just below where you set up your motor. (line 114 in default code)

- Add the line “setInvertedMotor(/* YOUR MOTOR PORT’S NUMBER HERE (like 5) */, true);

## Adding in an extra joystick-> motor Or servo control

- You need to tell the computer what you will be calling the motor (set up the object) – in this example the motor is called m_arm

- Set up the Joystick  -- in this example, the joystick is called m_operatorStick

- Then add the line “m_arm.set(m_operatorStick.getY());” in TeleopPeriodic (line 225 -294)

  - If you aren’t using the Default code, add it just below your drive code

- **If you want Servo control the line would be “m_arm.set((m_operatorStick.getY()+1)*.5);”

  - Servos have a range of 0 to 1, but joysticks and motors have a range of -1 to 1.

    - The general formula for converting a number from one range to another is “newValue = newMax +(newMax-newMin)*(oldValue-oldMax)/(oldMax-oldMin)” (Taken from the Chief Delphi post by FRC4ME in the thread “Servo Control with a Joystick”)

# Downloading your code

- Open up your Netbeans project and turn on your cRIO.

- Connect your computer to the cRIO (port 1) via a crossover Ethernet cable. (You can download wirelessly, but there can be complications. For now, tether the robot)

- Save your code. Now would be a great time to make another version of your code.

- Right click on your project name and click “Set as main project.” This will make the project name bold.

- Check to make sure that you don’t have any red stop signs with explanation points () in your code. (They will be located with the line numbers)

  - If you do go to Code Errors for more information.

    - You should turn off the cRIO to save power, and debug.

- Then click the green run button (or press F6).

  - If you get a “BUILD FAILED” message, there is something wrong.

    - If the last lines that are in black on the build output are “Connecting FTP @10.xx.yy.2” (remember that xx.yy will be replaced with your team number) and it says “Java.net.SocketException: Connection Reset” then you are not connected to the cRIO. Make sure the cRIO is on, and the Ethernet cord is in port 1 of the cRIO and in your Ethernet port on the computer.

- Open up the Driver Station (press windows+ L and click on the Driverstation icon on the classmate) and test your code.

# Set up your radio/Wireless Network

This information is available on the FIRST site. The link is http://usfirst.org/uploadedFiles/Community/FRC/Game_and_Season__Info/2010_Assets/2-2010FRCControl%20System-Getting%20Started-Rev-0.7.pdf

FIRST 2010 FRC Control System Manual: Section 2 Page 45 – This manual will be a huge resource for you. I highly recommend you download the pdf.

# Basic Vocab

## Robot vocab

#### Accelerometer

Accelerometer and Gyro

A sensor that can detect acceleration including the pull of gravity. The one we get in the kit is a 3 axis, so we can detect if we are tilting.

#### Analog Sensors

These plug into the Analog Breakout board. They give back values based on the intensity of what they are measuring (gyros, accelerometers, potentiometers, etc.)

#### Arcade driving

1 joystick controls the drivetrain. Pushing it forward makes the robot go forward, pushing it to the left makes the robot go left. (Generally people are better at first at arcade, but they get better at tank with only a little bit of practice)

CIM

CIM

the most common motor for the drive train. You can only use a limited (4) of these per robot

#### cRIO

Digital Sidecar

cRIO

the brain/main computer of the robot. Has many modules.

#### Digital Sidecar

This is where you plug in most of your wires (motors, servos, relays, LEDs and digital sensors)

#### Encoder

a digital sensor that can be used to measure rotations (and distances) (plugs into the digital I/O on the digital sidecar)

#### Fisher Price

Small highspeed motor. Comes with big plastic gearbox.

#### Gyroscope (aka Gyro)

analog sensor that can measure turning (of the robot) (can be used on Omni-bots to make pushing the stick forward always make the robot go away from you) – picture with the Accelerometer.

#### Jaguar

Jaguar

Jumper

A variable speed motor controller. (larger size than the Victors, and stylized) Plugs into the PWM OUT on the Digital sidecar)

#### Jumper

tiny piece of plastic covered metal – connects 2 pins. Used on the Analog Breakout board for getting battery voltage, and for servos.

#### Limit Switch

a digital sensor used to see if something is touching a certain point.

Limit Switch

Potentiometer (aka pot)

an analog sensor (plugged into the analog breakout board) that will give different values for different angles – but has a set range of motion.

#### PWM

3 colored wire (usually white-red-black) this will be your most of the wires coming out of the cRIO. It’s used to connect speed controllers to the Digital side cat and sensors to the cRIO

#### Spike

Spike

an on/off power switch for a motor (a relay). The PWM plugs into the relay ports on the digital sidecar.

#### Servo

a small motor with a built in potentiometer. Has a set range of motion. Plugs into the PWM OUT on the digital sidecar but a jumper must be added on the 2 pins next to it.

#### Tank driving

Uses the y axis of 2 joysticks to drive the robot. Each joystick control half of the drivetrain. Pushing the left stick forward will turn on only the left motors, and send the robot to the right. Pushing both sticks forward will make the robot go forward.

#### Victor

a variable speed motor controller. (smaller size than the jaguars, and more block-like) Plugs into the PWM OUT on the Digital sidecar

Window Motors

Window Motor

slow powerful motor. Usually runs off a relay.

## Programming vocab

#### Camel Case

the convention(tradition) of writing the first word in a series lower case, then the first letter of each following word uppercase. itShouldLookLikeThis

### Comparisons

There are two ways equal signs are used. (a=b) will set a to the value of b, while a == b will check to see if the a and b are equal. (a==b) will return a Boolean value of true or false.  Possible error message “Incompatible types” – you don’t have the same type of variable being compared. Possible warning:  “Possible loss of precision” This means 2 things. 1. You are setting a more precise (double) to a less precise one (int). 2. You are setting a to have the value of b, not testing if a and b are equal

! – Explanation points are the programming “not” (a!=b) will return true if  a doesn’t have the same value as b. It inverts the value of the Boolean. (A value that would be false returns true, and vise versa)

a>=b –If a is greater than or equal to b, it will return true

a<=b – If a is greater than or equal to b, it will return true

a>b  --If a is greater than b, it will return true

a<b – If a is less than b, it will return true

Keep in mind that (a>=b) will be the functionally the same as !(a<b)

#### Constants

In Java, variables and constants are created almost the same way, except a keyword “final” is put in front of the variable type. Traditionally, constants are named in ALL_CAPS and they cannot have spaces in the name.

final double PI = 3.14159265358979323;

#### Comments

Comments don’t run on the robot, they only help the programmer. You can comment out non-functional code or write a comment to describe a line. There are 2 types of comments: single line comments and multiline comments. A single line comment starts with a double forward slash, and comments out the rest of the line. A multiline comment starts with a forward slash then an asterisk (/* ) and ends with an asterisk and a forward slash (*/).

Not a comment   // this is a comment                         so is this

This isn’t

/*this is a comment                                                                           Still a comment

Still a comment */                   but this isn’t

#### Constructor

This is what you use to create an object. It’s what you use to tell the robot what you are calling something and where it is plugged in. See Setting up your own motors, sensors, and constants(page 15) for instructions on how to use it.

#### French Braces {}

They package code. They enclose the code executed during a loop or an if statement. They always come in pairs, make sure you have one on each side of your block of code. Possible error: Illegal start of Expression or Parsing error. Both mean you have the wrong number or placement of Braces. It’s often helpful to print out your code and draw lines connecting corresponding braces to see where you went wrong

#### Function/Method

A set of lines of code that can have parameters and return a value. It’s a good idea to make a function when you want to have a bit a code you call (or do) repeatedly.

There are many built-in functions (such as tankDrive(left

#### Objects

you don’t need to know very much about Objects, but you have to be able to create and use an object. You will be setting up your motors, relays, servos and everything through objects.

When you want to change or get information (through a function) you put the object name, then a period then the function (with all parameters)

#### Parameters

The inputs in a function. Think of it as the information it needs to know before it can do anything. (Like doing a homework assignment, you need to know the page it’s on before you can actually do it). Parameters are separated by a comma.

#### return

The value that a function or method gives back. 2+2 returns 4, 1==1 returns true

#### Semicolons

Semicolons are the periods of Java. They finish a statement.

#### Syntax

Java is a language, and syntax describes the grammar. If you get a syntax error, there is something wrong with the way you set up your line of code. In more depth in the Syntax section (here)

#### Variables

The value of this can change throughout the program

Boolean

only true or false

Double

any number, includes decimals

Int

almost all integers(no decimals allowed). Odds are high that the only time you’d need a greater range of numbers is if you were using a high resolution encoder. You would use a “long” in this case.

String

a “string” of numbers, letters and symbols. You are unlikely to use this variable type except in System.out.println(toPrint)

### Structures and Functions

Conditional – this is the statement that will be evaluated – it will have a Boolean (true/false) value, and can use Comparisons  such as =, >=, >, <=, < or !=.

If Statements – The most important conditional.  You can have if’s without an else or an else if, but not the other way around. You’ll get a syntax error if you try.

| if (/* INPUT WHAT YOU WANT TO BE TRUE */) { // THIS IS WHAT WILL HAPPEN if it’s true } else if(/* Second thing to test*/) /* Will only be tested if the original condition is false*/ { /* Will happen if the first condition is false, and the second is true*/ } else /* Notice that this doesn’t have a condition */ { // will happen if both conditions are false } | if(a>b) { System.out.println(a); //prints the value of a } else if(a<b) { System.out.println(b); //prints the value of b } else { System.out.println(“a and b are equal to “ a); } /*This will print out the value of the greater of the 2 variables, or state that they are equal and give the value*/ |
| --- | --- |

While loops. – the most important loop. It will execute the code within the French braces while the condition is true

| while(/*conditional is true*/) { // this code will be done while the conditional is true. } | while(isEnabled()) /* this code will run only while the robot is enabled ---- IT WILL ALSO ONLY RUN IF THE ROBOT IS ENABLED WHEN THIS CODE IS RUN (not if you enable later) */ { m_robotDrive.tankDrive(m_leftStick, m_rightStick); } // the above line is straight from the Default code |
| --- | --- |

# Calling a Function

Functions can have parameters, and you need to include the right data type for each. – there is a list of the functions and it is found at http://livemyst.comli.com/wpilibj/doc/index.html and the ones you are most likely to use are found below (with a brief explanation)

| Type | Returns | Function name | Parameters |  | Notes |
| --- | --- | --- | --- | --- | --- |
| private public | int double string boolean void | ExampleFunction( | int First, double Second); ); |  | If it returns void, it doesn’t give any value back. Don’t try to set it equal to something. |
| public void set(double speed); |  |  |  | You use this one by typing in the name of your motor (example here is m_shoulder) Range of values is -1 to 1 m_shoulder.set(0); // Stops the shoulder motor Somewhere above in your code, you must have initialized m_shoulder. See Setting up your own motors, sensors, and constants |  |
| public void set(double value); public void setAngle(double degrees); |  |  |  | Used for setting the position of a servo. (The set function acceots values from 0 to 1) If the servo has a max angle of X and a min angle of Y, values greater than X will result in X being set and angles less than Y will result in Y being set |  |
| public double get(); |  |  |  | You use this function to get the speed of the motor specified. Notice there are no parameters. speed = m_shoulder.get(); |  |
| public boolean get(); |  |  |  | Gets a digital input from cRIO (Must have set up m_frontLimit and hit) hit=m_frontLimit.get(); |  |
| public boolean getDigitalIn(int channel); |  |  |  | Gets a digital input from THE DRIVERS STATION final int SHOOT_BUTTON = 2; boolean shoot = false; shoot=m_ds.getDigitalIn(SHOOT_BUTTON); |  |
| public int getValue(); |  |  |  | Get an analog input from the Robot. Unless you are using a custom sensor, there are function calls for most sensors. turning =m_analogCustom.getValue(); |  |
| public double getAngle(); |  |  |  | Gyro – returns the heading heading = m_gyro.getAngle(); |  |
| public double getRangeInches(); public double getRangeMM(): |  |  |  | You set up at the which units you use at the beginning. distance = m_frontUltrasonic.getRangeInches(); |  |
| public void set(int value); |  |  |  | Setting the direction on a relay. The values for direction you can use are Value.kOn Value.kOff Value. kForward and Value.kReverse -- you can only use Value.kOn if the relay is set to go in only one direction //make sure you import edu.wpi.first.wpilibj.Relay.*; |  |
| public void tankDrive(double leftSpeed, double rightSpeed); |  |  |  | Allows you to drive tank style. if you named your robotDrive object m_robotDrive and you create and set leftSpeed and rightSpeed to your axises, you will call it by writing m_robotDrive.tankDrive(leftSpeed,rightSpeed); |  |
| public double getAxis(int axis); |  |  |  | This is used to get a joystick axis. you can use AxisType.kY, AxisType.kX, AxisType.kTwist, AxisType.kThrottle, and AxisType.kZ if you import edu.wpi.first.wpilibj.Joystick.AxisType; |  |
| public void arcadeDrive(double speed, double curve, boolean spin); public void arcadeDrive(GenericHID stick); |  |  |  | Used with RobotDrive 1.Speed is usually the y-axis of the stick and curve is the x. If you want the robot to be able to spin in place(zero point turn) spin should be true 2. Just set up the joystick, and input it (the object) here |  |

By writing the name of the thing(motor, joystick, sensor etc.) then putting a period, and then the function name, you are telling it which motor to act on.

# Tabbing

The easiest way to tab is to just indent with each opening French Braces and decrease the indent with the each closing French brace

if(/*conditional*/)

{

// do stuff

while(/*conditional*/)

{

// Do other stuff

if(/*conditional*/)

{

//Think about life, the universe, and everything

}

else

{

//Say 42

}

}

// Finish off your stuff

}

There is tool bar up top with a whole bunch of pictures, two of them have to do with tabbing your code. They are the ones that have a whole bunch of lines with a blue arrow.   The arrow points in the direction the indent will go.

# Basic Custom Control (Joystick to Motor)

You create a new project (Simple Robot)

There are 2 functions within the simple robot project: autonomous and operatorControl.

On line 11, there is the import line.

You will want to include (at least) these files See

import edu.wpi.first.wpilibj.SimpleRobot;	/* this one is included when you open the project. Don’t delete it */

import edu.wpi.first.wpilibj.Watchdog;		/*This is the watchdog. It is a safety feature that shuts off your robot if it can’t see the driverstation*/

import edu.wpi.first.wpilibj.Timer;

import edu.wpi.first.wpilibj.RobotDrive;		/*allows you to use pre-made functions for driving*/

import edu.wpi.first.wpilibj.Joystick;		/*allows you to get and use data from joystick */

import edu.wpi.first.wpilibj.Joystick.*;		/* allows you to use kY and other axis */

import edu.wpi.first.wpilibj. Jaguar; 		/*  allows you to control your motors---If you are using Victors instead of Jaguars, this line should say  “import edu.wpi.first.wpilibj.Victor;”   **** IF YOU ARE USING BOTH VICTORS AND JAGUARS, YOU NEED BOTH LINES*/

You need to tell the robot what you are calling everything and what type of thing it is.

To do that, go to the line after “public class FromScratch extends SimpleRobot { “ **NOTE: I named my class FromScratch. Your name will probably be different. Whatever you put in the “Robot Class” when you named the code will be in place of FromScratch** and press enter a couple times. This is where you should set up your constants and robot parts.

RobotDrive m_robotDrive     =new RobotDrive(1,2); /*Set for 2 motor drive, Left = port 1, right = port 2*/

Joystick m_leftStick	 = new Joystick(1);       /*joystick called m_leftStick plugged into Port 1*/

Joystick m_rightStick 	= new Joystick(2);	   /* joystick called m_rightStick plugged into Port 2 */

Joystick m_operatorStick	 = new Joystick(3);   	   /*joystick called m_operatorStick plugged to port 3*/

Jaguar   m_arm	= new Jaguar(3);	   /*joystick called m_arm plugged into port 3*/

double arm_speed 	=0;			  /* this is a variable- double (decimal values)  called arm_speed*/

If you haven’t edited it at all yet(I’d have to wonder why you are reading this if you haven’t), operator control starts on line 31, but once you have put in the imports, and you have set up your motors, it will be on a different(later) line. Go to the line after “public void operatorControl(){ “ and make a while loop.

while(isOperator Control() && isEnabled()) 	/*here’s a great time to get started on TABBING*/

{

Watchdog.getInstance().feed();

m_robotDrive.tankDrive(m_leftStick, m_rightstick);		// Drive tank style

// Now I am going to hook up a motor to a joystick. (In this case, the y axis.)

arm_speed = m_operatorStick.getAxis(AxisType.kY);  /*if you want the x axis, replace kY with kX*/

m_arm.set(arm_speed);

Timer.delay(.005);		//Don’t want the loop to run too fast

} // end while Operator control

# Creating your own Drive code

There are several main types of driving that most teams use. For basic drivetrains, there is tank or arcade. There are also omni drive systems, the most common being Mecanum (a type of wheel). All of these types have premade code for driving if you use robot drive.

Sometimes your motor would run in the opposite direction that it should, so you need to invert it.

Constructors:

RobotDrive(int leftMotorChannel, int rightMotorChannel)

RobotDrive(int frontLeftMotor, int rearLeftMotor, int frontRightMotor, int rearRightMotor)  /*It doesn’t really matter what motor is in front and which one is in the back */

These functions are set up the same way as shown in Calling a Function

public void arcadeDrive(double moveValue, double rotateValue, boolean spin);

public void arcadeDrive(GenericHID stick);	/*if you set it up as a joystick, it will still work here */

public void drive(double speed,  double curve);

public void holomonicDrive(double magnitude, double direction, double rotation); /*Holonomic drive is Omni drive for the Mecanums */

public void setInvertedMotor(int motor, boolean isInverted);

public void tankDrive(double leftSpeed, double rightSpeed);

public void tankDrive(GenericHID leftStick, GenericHID rightstick);

If you want to have a drive set up different than any of these (for example, crab drive) you will want figure out how each of the motors will be getting their information. Crab drive has one (or more) motors which turn the individual wheels. You would probably want an encoder or potentiometer on these, so you can easily “zero”(reset to straight) the wheels.

# Errors

## Syntax

Semicolons: Java :: periods:English

- If you have a generic syntax error, check the line and the one before it for a semicolon or other ending statement (like the correct amount of closing parenthesis or brackets)  -- Netbeans is pretty good at telling when the error occurs

- “Possible loss of prescision” – you are setting a more precise (like a double) to something less precise(like an integer). This is a warning, not an error. If you do set a double to an int,  you will lose all the numbers after the decimal. Ex int a =0; double b =5.9999; a=b;

- After executing the above code, a would be 5. If you want to get rid of the warning message, (but beware that you are doing the same thing) you cast the value.

  - The line would then look a = (int) b;  (a will still be 5. If you want to round the value, add .5)

- Parsing error  either “Class or interface expected” or “Illegal start of expression”  and if you have it set as your main file, and you press the Hammer button(Build Main Project (F11)) the last error will be “Reached end of file while parsing”

  - Each of these errors means your brackets aren’t in the right spots or you don’t have the right amount of them. NetBeans has a really cool feature, if you select(or place the cursor next to it) over one bracket or parenthesis, it will highlight the matching side of it.

# Setting up your own motors, sensors, and constants

Objects are set up in the same way every time. Up near the top, you need to include the corresponding file. (import edu.wpi.first.wpilibj.ObjectType;)

Then you decide what you are going to call it, write a line like ObjectType name;  This will be included after the line that starts “public class” and ends with a {  but before the constructor/initialize function

Then within the constructor/initialize function  you will add the line “name = new ObjectType(/*parameters*/);

In the default code, they set up a convention that your robot parts are prefaced with m_ and then written in Camel Case. So the left motor is usually called m_leftMotor

## Motors

Find out which speed controller you are using (Victor or Jaguar) and go to the correct section

#### Jaguars

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Jaguar;”

In this example, the motor is named m_jag and it is in port 7 – “Jaguar m_jag;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_jag = new Jaguar(7);”

#### Victors

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Victor;”

In this example, the motor is named m_vicky and it is in port 8 – “Victor m_vicky;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_vicky = new Victor(8);”

#### Servos

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Servo;”

In this example, the motor is named m_twitchy and it is in port 6 – “Servo m_twitchy;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_twitchy = new Servo(6);”

NOTE: Servos need a jumper on the 2 pins next to the PWM port on the Digital sidecar.

## Relay

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Relay;” and “import edu.wpi.first.wpilibj.Relay.*;”

In this example, the relay  is named m_blinky and it is in port 2 – “Relay m_blinky;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_blinky = new Relay(2, /*DIRECTION*/);”

Direction will be one of 3 things : Direction.kForward OR Direction.kReverse OR Direction.kBoth

If you only need to go one direction, set it as either Direction.kForward OR Direction.kReverse and then use Value.kOn OR Value.kOff . It makes it simpler

If you need to go both directions, you will have to use Value.kForward and Value.kReverse

## Sensors

#### Limit switches

These are Digital Inputs.

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.DigitalInput;”

In this example, the limit switch is named m_magLimit and it is in port 1 – “DigitalInput m_magLimit;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_magLimit = new DigitalInput(1);”

The constructor is DigitalInput(int channel)

#### Quadrature Encoder

This is 2 encoders, slightly out of phase with each other so direction can be sensed.

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Encoder;”

In this example, the limit switch is named m_clicky and it is in ports 2 and 3 – “Encoder m_clicky;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_clicky = new Encoder(2,3,false);”

The constructor is(int aChannel, int bChannel, boolean reverseDirection)

#### UltraSonics

This is one digital input, and one digital output.

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Ultrasonic;”

In this example, the ultrasonic  is named m_screech and the output is in port 3 and the input is on port 7, units are millimeters – “Ultrasonic m_screech;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_screech = new Ultrasonic(3,7, 1);”

The constructor is Ultrasonic(int pingChannel, int echoChannel, int units)

#### Gyro

This is an anolog sensor.

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Gyro;”

In this example, the relay  is named m_spinny and it is in port 2 – “Gyro m_spinny;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_gyro = new Gyro(2);”

The constructor is Gyro(int channel)

#### Potentiometers

This is an analog sensor

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.AnalogChannel;”

In this example, the potentiometer  is named m_pot and it is in port 1 – “AnalogChannel m_pot;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_blinky = new AnalogChannel(1);”

The constructor is AnalogChannel(int channel)

## Pneumatics

#### Solenoid

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Solenoid;”

In this example, the solenoid is named m_kicker and it is in port 2 – “Solenoid m_kicker;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_kicker = new Solenoid(3);”

The constructor is Solenoid( int channel)

#### Compressor

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Compressor;”

In this example, the compressor  is named m_compressor and it is in relay 3, and digital input 1 – “Compressor m_compressor;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_compressor = new Compressor(3,1);”

The constructor is Compressor(int pressureSwitchChannel, int compressorRelayChannel)

You will want to start (function is public void start();) in the initialize/constructor just below where you have the line “compressorName = new Compressor(cutOffPort, relayChannel);”  -- your start line should say “compressorName.start();”

## Driver Station

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.DriverStation;”

In this example, the driverstation is named m_ds– “DriverStation m_ds;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_ds = DriverStation.getInstance();”

THIS ONE IS NOT AN OBJECT –(no constructor)

## Joysticks

You’ll need to add the import (at top of code) “import edu.wpi.first.wpilibj.Joystick;”  and “import edu.wpi.first.wpilibj.Joystick.AxisType;”

In this example, the joystick  is named m_operatorStick and it is in port 3 – “Joystick m_operatorStick;”  --- this line will be put after the class, but before the constructor/initialize function

In the Constructor/initialize function, add “m_operatorStick = new Joystick(2);”

The constructor is Joystick(int port)

## Variables

Choose which type of Variables you want (Double, Int, String, or Boolean)  (also usually in Camel Case)

Then it is set up

variableType name;  //--- UNINITIALIZED. BAD Choice.

variableType name = startingValue;

ex.

double price = 1.23;

int count = 0;

string toPrint = “Hello world!”;

boolean fun = true;

## Constants

Set up as a variable with the keyword final (usually in all caps)

So it would be

final double PI =3.14159;

# Pneumatics

## Starting a Compressor

You need to have a couple of things plugged in for the compressor. It runs off of a relay, and it has a pressure cutoff switch.

The Constructor is Compressor(int pressureSwitchChannel, int compressorRelayChannel) See Setting up your own motors, sensors, and constants (page15)

You will want to start (function is public void start(); ) it in the initialize/constructor (just below where you have the line “compressorName = new Compressor(relayPort, cutOffPort);”

## Using A Solenoid (Using Pneumatics)

Plugged into the solenoid breakout board (slot 8 of the cRIO)

The constructor is Solenoid(int channel)

The Function is public void set(boolean on);

# Basic Steps to Problem Solving

- Recognize it can be done

  - “Impossible only means that you haven’t found the solution yet.” (from thinkexist.com)

- Think about the way you would do it if you had only the inputs you can give a robot

Ex. If you need to figure out how to follow a wall, close your eyes, and think about how you would do it blindfolded. (trail your hand along the wall…)

- You CAN make there be more sensors on the robot. It’s okay if you can’t do it with what you are given.

- Think about how the robot will have respond to its inputs and its limitations

- If you want it to go in a circle when a button is pressed, make sure it can physically go in a circle.

- Make sure that you only set your motors one time per loop. If you set it several times, you will be confused and your bot won’t work right. – which is why it is so important to use good variable names and constants.

Psuedo-code is my favorite coding tool. You just write out the main points of your program - don’t worry about syntax or how to read a sensor. Or even how you will implement something big. Get down order and choices. Then move on to the next piece that you skipped over. NOTE: It’s usually scribbled on random pieces of paper. It just helps you figure out what to do.

/*PSUEDO_CODE */

if(button)

circle

else

drive normal

- If you can’t figure it out (whether a small piece of code or the actual problem) ask for help. You can post on Chief Delphi or on FIRST Forum.

- I make models, walk through hallways and wave my arms around sometimes when I am trying to figure things out. Don’t feel stupid if you want to put yourself in your robot’s “shoes.”  It’s a helpful technique.

- Although timing is the technique most people fall back on (for autonomous coding) and it works and it works decently. If you want a more reliable technique (where the battery voltage won’t mess up your timing) try something sensor based.

# How to use a timer to wait to execute code (So you can keep driving)

This is a more advanced topic. It assumes that you understand drive code.

You will have to add “ import edu.wpi.first.wpilibj.Timer;”  and initialize and start a timer. (it’s a good idea to call it something relevant to why you need one ex. timeGrab)

Instead of using a wait function to wait for your delay, you can use a timer and if statements.

The code below also shows how we tackled the problem of doing something ONLY when the limit switch was first pressed.

/* This was actual code from 2234’s robot in 2010. We needed to add in a delay because we would push forward with pneumatics, but the string we were trying to latch onto with a compound bow trigger would move slightly. Waiting for250 ms made the grip much better

the servo is called m_trigger.

SHOOT is the input (and a constant) on the Cypress FirsTouch board that our button was connected to

RELEASE and GRAB are constants that we found through experimentation.

delay_grab is the name of the timer

m_canReach is the name of the limit switch (which tests to see if we are in reach to grab the string)

I’ve added the above comments, and elaborated slightly on the line “if(!m_ds.getDigitalInput(SHOOT))” The comment originally said “this switch defaults to true”. I also changed some of my names to keep with the naming convention shown here but those are the only things I changed.

NOTICE – tabbing, comments, readable names

*/

if(m_canReach.get())

{

if(!m_ds.getDigitalInput(SHOOT)) /*this particular switch defaults to true (this tests if the switch is pressed)—“if(!m_ds.getDigitalInput(SHOOT))” is same as “if(m_ds.getDigitalInput(SHOOT) == false)” */

{

m_trigger.set(RELEASE);

}

else

{//shoot button not pressed

if(pressed==false)

{// this means the limit switch was just pressed

delay_grab.start(); // start counting

pressed = true;

}

else

{ // the limit switch has been pressed for a little while

if(delay_grab.get() >= TIME_DELAYED)

{

m_trigger.set(GRAB);

delay_grab.stop();

delay_grab.reset();

}

else // less than our set time

{

//Less than set time, so do nothing

}

} // end else the limit switch has been pressed for a while

} // end shoot button not pressed

}// end limit m_canReach is pressed

else // not touching, therefore the trigger should be open. And reset the timer.

{

m_trigger.set(RELEASE);

pressed = false;

delay_grab.stop();

delay_grab.reset();

}//end else m_canReach

You can use a similar thought process (do something if it is greater than a predetermined time, and something else if it is less) to do anything you need.  Just make sure that you stop and reset when you are done, and you start the Timer when you need to.

Using if statements means that your code won’t stop while you are waiting for your amount of time, but if you used a Wait(ms) statement, your code would stop running for however long- WARNING: Just because your code isn’t running, doesn’t mean your motors stopped. They keep going until they are set to 0!

# Controlling a motor with a Sensor

When would you need to do this? How about if you want to make a custom driver station, or you use a dial to control the speed of a kicker? What if you decided that you wanted to make the robot go fastest when it is bright out?

## Analog Sensor

#### Gyro

Normally you are trying to drive straight (in autonomous) with a gyro, the below code will do that.

//Assuming the gyro is called m_gyro, and it has been initalized and started – and it’s in a loop.

m_robotDrive.drive(.5, m_gyro.getAngle()/2);		// Drive at half speed, with half the gyro angle //as correction.

#### Accelerometer

If you are a beginner programmer, I highly doubt that you will need to use this sensor. With it, you can tell if your robot is leaning (we are given 3 axis accelerometers in the kit, and if you orient it so when it is stationary there is +1G of force on the Z axis, if there is force on either of the X or the Y axises, you know it is tilting)

#### Potentiometer

You need something in a certain position (arm to x height, flap to open, etc)

/* this code will probably be activated with the push of a button.

The potentiometer is called m_pot and the motor is m_motor. I am going at speed MOVE (final double) to DEST (final int) and allowing an error of ERROR (final int)*/

if(m_pot.getAverageValue()+ERROR) < DEST)

{ //outside of tolerance

m_motor.set(MOVE);

}

else if((m_pot.getAverageValue() –ERROR)> DEST)

{// outside of tolerance

m_motor.set(MOVE*-1);

}

else // within tolerance

{

m_motor.set(0);

}

#### Custom Sensor/ Generic

Light sensors, potentiometers, or anything you made custom falls under this category. Because I don’t know what type of sensor you will be using, I am just going to show how to read the sensor, and a few ways to make a decision based on your values

There are a couple of ways to read your sensors -

#### Coolest thing I saw – Combining Potentiometers on the DS with ones on the Robot

A lot of teams have difficulty when operating an arm because they aren’t sure exactly how it will move when they move the joystick. This team (I’m sorry I don’t know the number, but they were at Championships in 2007. I believe they were in the same division as 341) had an outstanding solution. They created a miniature copy of  their arm for the driver station. There was a potentiometer on each of the 3 joints of the arm. The operator would move the arm around, and the robot would respond by mimicking the movement.

I have not had the opportunity to test this code yet, but I hope it will work!

/*

potentiometers on the robot are called m_potShoulder, m_potElbow, and m_potWrist (set up as generic analog sensors),

the potentiometer ports on the DS are called PORT_SHOULDER, PORT_ELBOW, and PORT_WRIST.

The DS is called m_ds and the motors are m_shoulder, m_elbow, and m_wrist.

There is a dead zone of (final int) ERROR.

The speed I move the motor at is SPEED (final double)

this is a Function I created so I don’t have to copy and paste and make my code longer if you want to put it in your code, you would need to  put it after the ending brace of a function but before the one of the class ( so if you put it at the bottom, there would be 2 ending braces, put the code between those 2)

*/

public double mimick(int ds_potValue, int robot_potValue, double speed)

{

if((robot_potValue +ERROR) < ds_potValue)

{ //outside of tolerance

return speed;

}

else if((robot_potValue –ERROR)> ds_potValue)

{// outside of tolerance

return (speed*-1);

}

else // within tolerance

{

return 0;

}

}

// the below code would be in your operator control function

m_shoulder.set(mimick(m_ds.getAnalogIn(PORT_SHOULDER),m_ potShoulder.getAverageValue(), SPEED);

m_elbow.set(mimick(m_ds.getAnalogIn(PORT_ELBOW),m_ potElbow.getAverageValue(),SPEED);

m_wrist.set(mimick(m_ds.getAnalogIn(PORT_WRIST),m_ potWrist.getAverageValue(),SPEED);

## Digital Sensor

When would use a digital sensor to control a motor?

  - A button on Driver Station to make a motor run at a specific speed

  - You want the robot’s Arm to go to a Specific Height? (Combination See below)

  - Stop an arm or anything from going past its physical limit

  - Quadrature encoders

#### Limit Switch

Only want to let the motor go one direction(direction shown is positive- if you want it to go in the other direction ) if the limit switch is pressed

/* Assuming that this is also within a loop, the limit switch is called m_limit and the motor is m_motor and the speed the motor should run at is speed*/

if((m_limit.getDigitalInput() == true)&&(speed<0)) // if it’s pressed and it is being sent a negative value

{	// then you shouldn’t move

m_motor.set(0);  // don’t move

}

else // otherwise, you are okay.

{

m_motor.set(speed);

}

You could also write the above code as:

if((m_limit.getDigitalInput() == true)) // if it’s pressed

{	// then you should test the sign (+ or -) of speed

if(speed<0)

{

m_motor.set(0);  /* don’t move because it would just move the motor even farther into the sensor */

}

else

{

m_motor.set(speed);

}

}

else // otherwise, you are okay.

{

m_motor.set(speed);

}

#### Button On Driver Station

Button to a preset motor speed – useful for a kick wheel – if at this spot, it needs to go this far

/* This is set up to set the motor to whichever speed once the button is pressed. – It assumes that unless the button is pressed, you want to control the speed with the y axis of a joystick.

I have called the driverstation m_ds(object, set up as the driverstation usually is), the motor is m_kick(object, set up as all motors are), the button (slot number, therefore a constant (final int)) is PRESET_BUTTON, the speed we are setting it to is PRESET  (final double), and the joystick is m_joy */

if(m_ds.getDigitalIn(PRESET_BUTTON) == true)

{

m_kick.set(PRESET);

}

else

{

m_kick.set(m_joy.getAxis(AxisType.kY));

}

NOTE: if you want to combine it with a  potentiometer, you would replace the part within the if with the code shown for the potentiometer. But keep in mind that it would only move to the specified height while the button was pressed (and held) – if you want it to go to that height after you just pressed the button (and don’t have to hold it) you would do something similar to what is shown below

Series of buttons for different speeds on a motor – useful for a kick wheel – if you are at this spot, it needs to go this far

/* This is set up to set the motor to whichever speed once the button is pressed. It will continue at that speed until a different button is pressed.

If 2 buttons are pressed, the lower of the 2 speeds will be the one sent.

I have called the driverstation m_ds, the motor is m_kick, the buttons are (slot numbers, therefore a constants (final int)) is STOP_BUTTON, QUARTER_BUTTON, HALF_BUTTON and FULL_BUTTON, the speeds we are setting it to are STOP, QUARTER_SPEED, and FULL_SPEED  (final double) */

if(m_ds.getDigitalIn(STOP_BUTTON) == true)

{

speed = STOP;

}

else if(m_ds.getDigitalIn(QUARTER_BUTTON) == true)

{

speed =QUARTER_SPEED;

}

else if(m_ds.getDigitalIn(HALF_BUTTON) == true)

{

speed = HALF_SPEED;

}

else if(m_ds.getDigitalIn(FULL_BUTTON) == true)

{

speed =FULL_BUTTON;

}

/* Notice that there is not an else – I only want the value of speed to change if there is a button pressed */

m_kick.set(speed);

#### Encoder

Set up the encoder (shown in the Quadrature Encoder of the Setting up your own motors and sensors)

This can be used to measure distance traveled (if it’s on the drive wheel), the location of an arm, or speed of a wheel.

If you want it to measure the distance for you, there are 2 ways. Take the number from public int getRaw() and multiply it by wheel circumference(if direct drive) then divide by the number of pulses per rotation. On the other hand you could use public void setDistancePerPulse(double distancePerPulse) and use the public double getDistance() function

You can get speed by calling public double getRate()

You can get the location of an arm (or whatever else you want) with by getting the distance, and having angular units (degrees or radians) instead of linear ones. Then associate the angle with your height or whatever you end up measuring. If your thing can loop around several times, perhaps you want to reset it every time it hits a limit switch (will correct for any slippage as well).