Remove the “no-back” rollers from the transmissions ASAP.  The pins on carrier that engage the output shaft can easily be broken with the no-back rollers in place.

These pins break

## Shifting is trickier than you think -- I recommend against it.

## Only 1 team I know does it reliably (Delphi-Kokomo) & it takes time and some fancy programming

## Make “rings” to install INSIDE the transmissions to lock them in HI or LOW

## Lock out the clutch mechanism

## replace spring with length of tube OR

## tap the 6 holes with an M-6 Tap and replace the balls with M-6 set screws (My Favorite method)

## Reverse Engineer the output shaft/final carrier, make smooth long shafted replacement

## Use a Trantorque™ on the Extended Shaft (NOT on the threaded as FIRST recommends)

## Support the drill shaft using two bearings on the Extended Shaft – only use the drill housing to provide TORQUE not as a bearing for the shaft

## Alternative to making Extended Shaft: Make a hollow “Chuck-like” Shaft that screw onto the external threads of the drill shaft and allows a left handed screw to screw into the center of the drill shaft.

## Mixed results – can be difficult to install tightly.  Once installed, difficult to remove without damage.